# Duraudio
Audio Hardware-Specific functions

v 1.0

November 26, 2001

Revision History

| 11/26/01 - Version 1.0 | Initial Version by Mario Raposo |
| --- | --- |

#### Table of Contents

Duraudio Audio Hardware-Specific functions	1

Description	3

Initialization	4

DURAUDIO_Initialize	4

DURAUDIO_SetupVSA	4

DURAUDIO_InitAudioRegs	5

DURAUDIO_SetDMARegs	5

IO Access	6

DURAUDIO_IO_PCI_Read16	6

DURAUDIO_IO_PCI_Write16	6

DURAUDIO_IO_PCI_Read32	6

DURAUDIO_IO_PCI_Write32	7

DURAUDIO_PortWrite32	7

DURAUDIO_PortRead32	7

CODEC Access	8

DURAUDIO_CodecWrite	8

DURAUDIO_CodecDirectRegRead	8

DURAUDIO_WaitForNextFrame	8

DURAUDIO_GetValidData	9

DURAUDIO_SetCodecRate	9

PRD Control	10

DURAUDIO_StartDMA	10

DURAUDIO_ClearStat	10

DURAUDIO_InitPRD	10

DURAUDIO_SetupPRD	11

DURAUDIO_GetPosition	11

DURAUDIO_InterruptID	11

Debugging	12

DURAUDIO_SetDebuglevel	12

DURAUDIO_GetDebuglevel	12

DURAUDIO_DisplayCODECValues	12

Non-DirectSound Functions	13

DURAUDIO_SetupIRQ_DMA	13

DURAUDIO_WaveOpen	13

DURAUDIO_WaveClose	13

DURAUDIO_WaveStart	14

DURAUDIO_SetupDMA	14

DURAUDIO_FillBuffer	15

DURAUDIO_FillDMABuffer	16

DURAUDIO_FillUserBuffer	16

DURAUDIO_WaveContinue	16

DURAUDIO_WaveStop	17

DURAUDIO_Standby	17

DURAUDIO_GetVolume	18

DURAUDIO_SetVolume	18

DURAUDIO_Free	18

DURAUDIO_GetInterruptType	18

1. Overview/General Description

Duraudio is a group of hardware specific functions that define a general interface to be portable to other Operating systems and to run on the National Semiconductor / Geode Platforms Cyrix MediaGX CPUs.

These functions include interfaces to an AC'97 Codec, Hardware Initialization and DMA buffer input/Output.

Duraudio files:

5530_def.h - Contains the definitions used by Duraudio.

5530_glb.h - Contains the Global variables and structures

OS_inc.h - Contains the OS-Specific definitions/Macros. The driver developer has to define these macros according to the O/S to which the driver is being developed.

2. Description of Features

Duraudio isolates the important hardware calls from the driver code.  That makes any changes in the hardware code be common to all the drivers in all opearting systems.

3. Installation Instructions

There are no specific installation instructions for Duraudio.  Duraudio is compiled as part of the driver and the installation instructions should be the same as the driver in the specific operating system.

4. Test Plan

There is no specific test plan associated with this set of functions. Duraudio is not tested separately from the driver code.  The test plan should be the same as driver’s test plan.

5. Design

## Initialization

These are functions related to the setup of the hardware and initialization of VSA for Audio streaming.

### DURAUDIO_Initialize

Initializes the Geode hardware.

unsigned char DURAUDIO_Initialize
( 
 	unsigned char Irq, 
 	unsigned short IoBase
)

It has to be called with the resources that were assigned by the OS

This function:

- Sets VSA audio irq component to create SMI generated interrupts Call to SetupVSA

- Resets and initializes the CODEC

Resources assigned by the system:

unsigned char   Irq		- Interrupt allocated to the device.

unsigned short  IoBase	- I/O base for the device.

In the case of windows drivers, these resources are assigned by the plug-and-play system.

### DURAUDIO_SetupVSA

void DURAUDIO_SetupVSA (unsigned int Irq)

Initializes the VSA.  This code initializes both VSA types:  VSA1 and VSA2.

It sets up the VSA to generate Interrupts based on the SMI generated when the BUS master finds a EOP flag in a PRD table entry.

### DURAUDIO_InitAudioRegs

Initializes the audio registers.

unsigned long DURAUDIO_InitAudioRegs (unsigned int IoBase)

- Probes for PCI device register spaces

- Checks to see if the Geode or 5530 Bridge header is found.

- Gets back the Vendor_ID+dev_id reg (1st word in PCI Header).

- Detects what kind of CODEC is being used (LM45XX or Analog devices).

- Maps the memory, if necessary.

- Resets the CODEC

- Intialize the CODEC with default values.

### DURAUDIO_SetDMARegs

This function:

unsigned char * DURAUDIO_SetDMARegs 
(
 	unsigned long 	PRD_phys, 
 	WAPI_INOUT 	Direction
)

- Clears the correct EOP bit according to the direction of the stream specified by Direction

- Copies the physical address of the PRD into the PRD address register.

- Returns a pointer to the Start/Stop register based on the direction of the stream indicated by Direction.

## IO Access

### DURAUDIO_IO_PCI_Read16

Reads 16 bits to the PCI I/O space

unsigned short DURAUDIO_IO_PCI_Read16
( 
 	unsigned long 	pdev, 
 	unsigned short 	port
)

### DURAUDIO_IO_PCI_Write16

Writes 16 bits to the PCI I/O space

void DURAUDIO_IO_PCI_Write16
(
 	unsigned long 	pdev, 
 	unsigned short 	port, 
 	unsigned short 	data 
)

### DURAUDIO_IO_PCI_Read32

Reads 32 bits to the PCI I/O space

unsigned long DURAUDIO_IO_PCI_Read32
(
 	unsigned long 	pdev, 
 	unsigned short 	port
)

### DURAUDIO_IO_PCI_Write32

Writes 32 bits to the PCI I/O space

void DURAUDIO_IO_PCI_Write32(unsigned long pdev, unsigned short port, unsigned long data)

### DURAUDIO_PortWrite32

Writes 32 bits to the PCI I/O port

void DURAUDIO_PortWrite32
(
 	unsigned short 	reg, 
 	unsigned long 	val
)

### DURAUDIO_PortRead32

unsigned long DURAUDIO_PortRead32 (unsigned short reg)

Reads 32 bits from the 5530/5540 I/O port

## CODEC Access

### DURAUDIO_CodecWrite

void DURAUDIO_CodecWrite 
(
 	unsigned short CodecRegister, 
 	unsigned short CodecData 
)

Writes data in CodecData to the register specified by CodecRegister.

### DURAUDIO_CodecDirectRegRead

Reads data from the regsiter specified by CodecRegister

unsigned short DURAUDIO_CodecDirectRegRead
(
 	unsigned short CodecRegister 
)

### DURAUDIO_WaitForNextFrame

void DURAUDIO_WaitForNextFrame(unsigned long val)

Waits for next avail pos in the CODEC's serial frame and sends the data.

While the CODEC_CMD_VALID bit is set, it should not be overwritten.

i.e., the codec cycles around to ship the command out at the next avail pos in the serial frame.

### DURAUDIO_GetValidData

unsigned long DURAUDIO_GetValidData(void)

This function waits for Status Tag and Codec Status Valid bits to be set and then returns new valid data from the CODEC.  If it times out waiting for these bits, we return what we have.

### DURAUDIO_SetCodecRate

Sets the CODEC Sample Rate based on Direction.

void DURAUDIO_SetCodecRate 
(
 	unsigned long Rate,  
 	Direction
)

## PRD Control

### DURAUDIO_StartDMA

Using the pointer returned by a call to DURAUDIO_SetDMARegs and the Direction, it Starts the DMA.

void DURAUDIO_StartDMA
(
 	unsigned long 	PRD_phys, 
 	WAPI_INOUT 	Direction
)

### DURAUDIO_ClearStat

Clear the status bit of the respective Audio Bus Master based on the Direction.

unsigned char DURAUDIO_ClearStat(WAPI_INOUT Direction)

### DURAUDIO_InitPRD

Initializes a PRD table. It fills n PRD Entries, where n is specified 
by NumberOfEntries with the size specified by BlockSize and flags=0.  The pointers in each entry are based on DMAPhys and incremented by BlockSize.
At the last entry, it puts the JMP flag and points to the top of the PRD using the PRDs physical address specified by PRDPhys.

void DURAUDIO_InitPRD
(
 	tPRDEntry		*pPRDEntry, 
 	unsigned long   	PRDPhys,
 	unsigned int	 	NumberOfEntries, 
 	unsigned int    	BlockSize,
 	unsigned long	 DMAPhys
)

### DURAUDIO_SetupPRD

void DURAUDIO_SetupPRD
(
 	tPRDEntry		*pPRDEntry, 
 	unsigned int		NumberOfEntries, 
 	unsigned int 	BlockSize,
 	unsigned long	SampleRate,
 	unsigned int		InterruptInterval
)

Setup the PRD entries with EOPs to generate interrupts based on the

Sample rate and the InterruptInterval value expressed in Miliseconds.

### DURAUDIO_GetPosition

Returns the position in bytes of the current data being played based on the current PRD address.

unsigned long DURAUDIO_GetPosition 
(
 	WAPI_INOUT 	Playback, 
 	tPRDEntry * 		pPRDEntry, 
 	unsigned long 	PRDPhys
)

### DURAUDIO_InterruptID

unsigned char DURAUDIO_InterruptID (void)

Returns the interrupt ID based on the copy of the status register made by VSA to 0x04F0 and clear the interrupt bits.

## Debugging

### DURAUDIO_SetDebuglevel

void DURAUDIO_SetDebuglevel (unsigned long Level)

Sets the debug level that is stored on F3BAR+4C

### DURAUDIO_GetDebuglevel

unsigned long * DURAUDIO_GetDebuglevel (void)

Returns the debug level that is stored on F3BAR+4C

### DURAUDIO_DisplayCODECValues

void DURAUDIO_DisplayCODECValues ()

Displays the values in the CODEC registers

## Non-DirectSound Functions

### DURAUDIO_SetupIRQ_DMA

unsigned long DURAUDIO_SetupIRQ_DMA (unsigned int Irq)

Sets up the IRQ and allocates the DMA buffers memory.

- Maps the Irg to a system interrupt number

- Allocates the 4 DMA Buffers

- Map the DMA pages into the local address space

### DURAUDIO_WaveOpen

Opens or query the wave stream.

MMRESULT DURAUDIO_WaveOpen 
(
    WAPI_INOUT 		apidir,                                           
    LPWAVEFORMATEX 	lpFormat, 
    unsigned char 		fQueryFormatOnly
)

It sets the g_pcmtype variable with

the right value to be used by the DURAUDIO_FillBuffer routine

and sets the sample rate.

PCM_TYPE_M8  - PCM Mono 8bit

PCM_TYPE_S8  - PCM Stereo 8bit

PCM_TYPE_M16 - PCM Mono 16bit

PCM_TYPE_S16 - PCM Stereo 16bit

It also can be used to query the format to see if it's a valid one.

To do so, the caller has to set fQueryFormatOnly to TRUE.

### DURAUDIO_WaveClose

void DURAUDIO_WaveClose(void)

Closes the Wave Stream. It sets the "In use" flag to FALSE and resets the digital volume to zero

### DURAUDIO_WaveStart

If the digital volume is 0 (nobody set it), set it to MAX.

void DURAUDIO_WaveStart
(
 	PWAVEHDR pwh
  	unsigned long dma_page_size
)

// Setup Flags for input and output buffers

//

//   v_nNextPage:  Points to the next buffer to be filled with data.

//   v_fMoreData:  Flag that signals if there is more data coming.

### DURAUDIO_SetupDMA

void DURAUDIO_SetupDMA (unsigned long dma_page_size)

Sets up the DMA pages.

The 5530/5535 contain an "audio buffered PCI bus master" that performs DMA-like functions.   The Physical Region Descriptor (PRD) specifies a buffer to be "DMAed" by the PCI bus master. The prd_array_phys_adr in the mem-mapped 5530 audio regs points to the start of an array of PRD descriptors.

Each entry of this array consists of 2 32-bit words:

32-bit word 0:   Address of the DMA buffer  .

1:   bit-31:   End of Table (EOT)

bit-30:   End of Page  (EOP)

bit-29:   Loop         (JMP)

bit-15:0  byte count.

In this function, the PRD table is initialized as following:

+----------------------------------------+

prd_array[0]  !       Address of the 1st buffer          +

+----------------------------------------+

prd_array[1]  !  EOP bit set  !  Length of 1st Buffer +

+----------------------------------------+

prd_array[2]  !       Address of the 2nd Buffer         +

+----------------------------------------+

prd_array[3]  !  EOP bit set  !  Length of 2nd Buffer+

+----------------------------------------+

prd_array[4]  !       Pointer to 1st PRD entry           +

+----------------------------------------+

prd_array[5]  !                JMP bit set                     +

+----------------------------------------+

### DURAUDIO_FillBuffer

Fills the Audio buffers according to the global variable GlobalApidir. GlobalApidir indicates the direction of the stream:

PWAVEHDR DURAUDIO_FillBuffer
(
 	PWAVEHDR 	pwh, 
 	short * 	pBuffer
)

Playback: GlobalApidir is equal to 1.

Record:   GlobalApidir is equal to 0.

pwh - Pointer to the WAVEHDR structure of the stream

pBuffer - Pointer to the DMA buffer to be Played/Recorded

If it's Playback,   it calls DURAUDIO_FillDMABuffer.

If it’s Recording, it calls DURAUDIO_FillUserBuffer

### DURAUDIO_FillDMABuffer

Fills the hardware DMA buffer with the data from the user's buffer to be played.

PWAVEHDR DURAUDIO_FillDMABuffer
(
 	PWAVEHDR 		pwh, 
 	signed short * 	pBuffer
)

pwh     - Pointer to the WAVEHDR structure of the stream

pBuffer - Pointer to the user buffer to be played.

It points to the data that will fill the Hardware buffer

### DURAUDIO_FillUserBuffer

PWAVEHDR DURAUDIO_FillUserBuffer
(
 	PWAVEHDR 		pwh, 
 	signed short  * 	pBuffer
)

Fills the user buffer with the data recorded (from the hardware DMA buffer).

pwh     - Pointer to the WAVEHDR structure of the stream

pBuffer - Pointer to the user buffer to save the recorded data.

It points to the buffer that will be filled with data

from the Hardware buffer

### DURAUDIO_WaveContinue

void DURAUDIO_WaveContinue (PWAVEHDR pwh)

This function is called with v_nNextPage[WAPI_OUT] containing the buffer that has just finished DMAing, and thus the buffer that is to be filled with data. The _other_ buffer is the buffer that needs to have its DMA activated.

### DURAUDIO_WaveStop

void DURAUDIO_WaveStop()

Stops the wave stream.

It zeroes the next buffer to be played and put a EOT

(End of table flag) on next PRD.

### DURAUDIO_Standby

void DURAUDIO_Standby(void)

{

DURAUDIO_ClearStat(GlobalApidir);

Running =FALSE;

fInUse =FALSE;

}

### DURAUDIO_GetVolume

unsigned long DURAUDIO_GetVolume(void)

Returns the digital volume to the caller

### DURAUDIO_SetVolume

Sets the digital volume based on the value sent by the caller

unsigned long DURAUDIO_SetVolume(void)

### DURAUDIO_Free

void DURAUDIO_Free(void)

This routine is called when the driver

is being unloaded. Frees the memory.

### DURAUDIO_GetInterruptType

AUDIO_STATE DURAUDIO_GetInterruptType(VOID)

Called by the driver's ISR.

Returns the state of the stream being Played/Recorded.