Why and Where multiplexing

Contribution to the MPEG/IETF meeting

meeting hosted by Columbia University

New-York, 25th of April 1999

D.Curet, C.Islas, K.Renout (FT)

# Introduction:

We will speak rather of two levels: the application level and the transport level.

# Abstract

This contribution points out the importance of application multiplexing mechanisms independent from the transport level.

Some enhancements to the application multiplexing tool supported by MPEG4 system are also proposed.

# Why do we multiplex at the application and at the transport level?

Pros:

- Multiplexing allows to reduce the number of managed transport channels, on both client and server side.

- The transport channel bandwidth can be shared among different programs or scenes.

- Application multiplexing can allow to adapt the application to an existing transport protocol, can allow to optimize the transport channel occupation. This multiplexing can be based on priorities and statistical characteristics of the different elementary streams.

- Application multiplexing can be done taking into account the buffer model occupancy.

- Application multiplexing allows to guarantee the same transport QoS for all the streams multiplexed within one transport channel.

- As streams multiplexed at the application level are transported within the same transport channel, they will be affected by the same network jitter. On the opposite, different streams transported over different transport channels will experience different network jitter over each transport channel.

- The overhead added by packet headers, in case of multiplexing, is generally lower than the overhead added by packet headers for separated streams. This allows also to optimize bandwidth resource allocation. The greater is the number of application-multiplexed streams, the lower is the transport overhead.

- Within the same application session, the number of elementary streams is changing dynamically. The transport channel establishment is inducing a delay for each new elementary stream. This delay does not exist when a new elementary stream is added to an application-multiplexed stream to which a transport channel has already been assigned.

- Conditional access is generally provided through conditional messages embedded in dedicated elementary streams, tightly coupled with the associated programs or scenes. There is a need also for service providers to associate such streams inside the same application multiplex. The example can be taken from scrambled contents: ECM messages (defined within DVB) carry control words, which are tightly coupled and multiplexed with the scrambled contents.

Cons:

- Multiplexing is adding a delay due to the multiplexing process.

- The loss of one or several transport packets of a multiplexed stream can be more severe than the loss of one or several transport packets of a non-multiplexed stream.

Neither Pros nor Cons:

- Transmission delays can be minimized when elementary streams are multiplexed, as opposed to multiple transport channels

- Multiplexing allows to reduce the number of threads, on both client and server side.

- The transport channel bandwidth can be spared among different programs or scenes.

- Connection management for separate streams can lead to some difficulties (memory management, CPU, security,..), when compared with the management of one multiplexed stream.

# What can provide multiplexing?

- The possibility to support “In band signaling” is one feature that must be offered by a multiplexing mechanism.

- Generally a “ sub-multiplexing mechanism ”  is    supported :

- When fixed length transport packets are used, it is indeed interesting to allow small segments of information to be transported without waiting for the completion of a complete multiplex packet with packets belonging to the same elementary stream.

- When variable length transport packets are used, it is interesting to group these small segments before sending them over the network.

To take examples:

- Within MPEG2, sections carriage inside transport packets support the above mentioned points 1 and 2a.

- RTP is supporting the 1 and 2b points.

# Where to multiplex?

As application is transport unaware, and as transport is application unaware, multiplexing is interesting at both application and transport levels.

The presence of multiplexing mechanisms at both levels can make use of different features supported by each level.

A multiplexing mechanism at the application level allows to compensate for transport protocols not supporting multiplexing, and a multiplexing mechanism at the transport level allows to compensate for applications not supporting multiplexing.

# Conclusion

We showed that multiplexing mechanisms at the application or at the Transport level cannot be done with the same knowledge of the transport network and of the media.    Multiplexing mechanisms at each level tackle different issues, and when they are provided, they are welcome.

# Proposal

As application multiplexing is supported by MPEG4 (the FlexMux tool), and as this tool is aware of the MPEG4 System Decoder Model, the transport of MPEG4 FlexMultiplexed streams over RTP should be specified by IETF.

# MPEG4 specification improvement: MPEG4’s FlexMultiplexing  and SL layer:

In its simple multiplexing mode, the MPEG4 FlexMultiplexing is adding a two-byte header to the embedded SL_PDU header:

- One byte for the elementary stream identification.

- One byte for the length.

In this simple mode, a FlexMux_PDU is two bytes longer than a SL_PDU.

The conclusion is that the embedded SL_PDUs shall be at most 255 bytes long when they are Flex_Multiplexed, whereas their length is not limited when they are not Flex_Multiplexed.

The sub_multiplexing mechanism (mentioned above in the paragraph related to “ What can provide also multiplexing, point 2a ”) is called MuxCode mode. According to the MuxCode mode, SL_PDUs of different origins can be concatenated inside only one FlexMux_PDU. In this MuxCode mode, the MPEG4 FlexMultiplexing process is defining a FlexMux_PDU with a three-byte header:

- One byte for the elementary stream identification.

- One byte for the length.

- One byte for the version (4bits) of the MuxCode table and for 4 reserved bits.

In this MuxCode mode, a FlexMux_PDU is three bytes longer than the sum of all the aggregated SL_PDUs.

A MuxCode table tells what is the length of the SL_PDU that is to be found as payload of the FlexMux_PDU, and which elementary stream they belong to. MuxCode tables are part of the signaling and are sent in advance.

The assumption is that Access Units have a variable length. As SL_PDU packetisation does not support byte padding (some bytes added to the payload to reach a certain number of bytes), it appears that the MuxCode FlexMultiplexing mode specification and the SL layer features may have some weak points.

Lets start from the example of one MPEG4 elementary stream supporting variable (and unpredictable) length Access Units (which is the common case):

When such an elementary stream is FlexMultiplexed using the MuxCode mode, all its SL_PDUs shall have a constant length. According to the MuxCode mode table semantics one value is associated to the length of each SL_PDUs of that elementary stream. However as Access Units are of variable length, the last SL_PDU embedding the last segment of the Access Unit should have a smaller length, as SL_PDU packetisation does not support byte padding (see the above mentioned Assumption).

Which means that before sending the last smaller SL_PDU, a new version of the MuxCode table has to be sent.

As the FlexMultiplexing mechanism is needed at the application level, if the SL layer is not supposed to support byte padding, is it possible (even not practical) to say that such elementary streams –with variable length Access Units- will never participate to any MuxCode mode multiplexing?

## 9.1 Conclusion on MPEG4 specification:

The FlexMux_PDU length issue, the sub-multiplexing mechanism, the SL_PDU padding mechanism should be revisited by MPEG4.

## 9.2 Proposal for the MPEG4 specification improvement:

FlexMux_PDU length: adopt a configurable length field as defined for descriptors.

If the MuxCode mode FlexMux_PDU syntax can be enhanced to suppress the MuxCode table, SL_PDU padding mechanism could be left as it is now.

If the MuxCode mode FlexMux_PDU syntax cannot be enhanced to suppress the MuxCode table, SL_PDU padding mechanism has to be modified (to support byte padding as MPEG2/PES support it)