SL layer functionalities

MPEG/IETF meeting

Meeting hosted by Columbia University

New-York, April 1999

D.Curet (FT)

# Introduction:

According to the current MPEG4 SL layer specification:

- It is possible to require a certain QoS for a complete elementary stream.

- It is also possible to indicate to a transport network channel adaptor the importance of a particular Access Unit of that elementary stream,

- It is impossible to indicate the importance of smaller part than the Access Units.

The contribution tries to highlight these points and is proposing a solution based on an extension of the existing SL_PDU header specification.

# Degradation Priority:

In the current MPEG4 SL layer specification, a field called “ degradationPriority ” appears in the SL_PDU header definition.

Such a field is not mandatory.

When used, it will appear in the SL_PDU header only when an Access Unit is declared to start within this SL_PDU. It indicates the importance of the payload of the starting Access Unit.

This indication can be used for graceful degradation by the decoder of this elementary stream.

The priority of this access unit is = streamPriority - degradationPriority.

The streamPriority being part of the Elementary Stream Descriptor.

It is indeed interesting that the SL layer could give some information on the importance of a particular Access Unit of an elementary stream, but as far as visual elementary streams are concerned, such information would be completely relevant if there were no smaller parts of an access unit that could be of some importance. All the configuration headers that can appear before each VOP can be felt to be more important than the VOP itself. Furthermore, the visual specification defines a self-decodable “ video packet ” structure which is a sub-part of a VOP.

There is no means in the current specification for the SL layer to indicate precisely (in the SL_PDU header) to the following transport channel adaptor such important embedded part, as they are smaller than an Access Unit.

The degradationPriority semantics can be extended to qualify smaller parts than an Access Unit. What the degradationPriority semantics could do for smaller part than Access Units would work also for the Access Units themselves.

# Conclusion:

The “degradationPriority” field could certainly of a better use for the transport network adaptor if its semantics were extended to the logical structures smaller than Access Units.

# Proposal:

- degradationPriority field enhancement:

The name can be maintained.

The degradationPrority semantics can be kept, as they are (only changing access units into logical structure).

In the SL_PDU header specification, the degradationPriority field should be taken out of the current if(accessunitStartFlag) {…..} statement, as it will appear eventually at each SL_PDU header.

- Definition of a normalised table called “Logical Elementary Stream structure ” :

For each streamType that table defines a numbered list of all the logical structures.

- Definition of a second normalised table called “ degradationPriority configuration ”:

According to the streamType (found in DecoderConfigDescriptor) a logical Elementary Stream structure is assigned (by the number found in the preceding table) to each degradationPriority value.

That degradationPriority configuration table will be part of the SLConfigDescriptor.