TCHRT

Turbo C High Resolution Timer Toolbox

Version 3.00

Shareware Evaluation Version

Ryle Design

P.O. Box 22

Mt. Pleasant, Michigan 48804

(517) 773-0587

Cserv 73047,1765

Contents

TCHRT User Guide                                           1

Introduction  . . . . . . . . . . . . . . . . . . . . . 1

Functional Overview . . . . . . . . . . . . . . . . . . 2

Theory  . . . . . . . . . . . . . . . . . . . . . . . . 3

TCHRT Function Descriptions . . . . . . . . . . . . . . 4

Examples  . . . . . . . . . . . . . . . . . . . . . . . 8

Technical Support . . . . . . . . . . . . . . . . . . . 8

Shareware . . . . . . . . . . . . . . . . . . . . . . . 8

TCHRT Function Reference                                  11

t_ask_delay . . . . . . . . . . . . . . . . . . . . .  11

t_ask_timer . . . . . . . . . . . . . . . . . . . . .  11

t_bios_ask  . . . . . . . . . . . . . . . . . . . . .  12

t_bios_entry  . . . . . . . . . . . . . . . . . . . .  12

t_bios_exit . . . . . . . . . . . . . . . . . . . . .  13

t_bios_fname  . . . . . . . . . . . . . . . . . . . .  13

t_bios_load_desc  . . . . . . . . . . . . . . . . . .  13

t_bios_report . . . . . . . . . . . . . . . . . . . .  14

t_bios_resume . . . . . . . . . . . . . . . . . . . .  14

t_bios_reset  . . . . . . . . . . . . . . . . . . . .  14

t_bios_rname  . . . . . . . . . . . . . . . . . . . .  15

t_bios_set_file . . . . . . . . . . . . . . . . . . .  15

t_bios_set_user . . . . . . . . . . . . . . . . . . .  15

t_bios_start  . . . . . . . . . . . . . . . . . . . .  16

t_bios_stop . . . . . . . . . . . . . . . . . . . . .  17

t_bios_suspend  . . . . . . . . . . . . . . . . . . .  18

t_calc_delay_ff . . . . . . . . . . . . . . . . . . .  18

t_calib . . . . . . . . . . . . . . . . . . . . . . .  18

t_cvt_time  . . . . . . . . . . . . . . . . . . . . .  19

t_delay_calib . . . . . . . . . . . . . . . . . . . .  19

t_diff  . . . . . . . . . . . . . . . . . . . . . . .  19

t_do_delay  . . . . . . . . . . . . . . . . . . . . .  20

t_do_delay_wints  . . . . . . . . . . . . . . . . . .  20

t_entry . . . . . . . . . . . . . . . . . . . . . . .  21

t_exit  . . . . . . . . . . . . . . . . . . . . . . .  21

t_fname . . . . . . . . . . . . . . . . . . . . . . .  21

t_get . . . . . . . . . . . . . . . . . . . . . . . .  22

t_get_delay_ff  . . . . . . . . . . . . . . . . . . .  22

t_hires_entry . . . . . . . . . . . . . . . . . . . .  23

t_hires_exit  . . . . . . . . . . . . . . . . . . . .  23

t_hook_int  . . . . . . . . . . . . . . . . . . . . .  23

t_min_delay . . . . . . . . . . . . . . . . . . . . .  24

t_name  . . . . . . . . . . . . . . . . . . . . . . .  24

t_report  . . . . . . . . . . . . . . . . . . . . . .  24

t_request . . . . . . . . . . . . . . . . . . . . . .  25

t_res_delay . . . . . . . . . . . . . . . . . . . . .  25

t_reset . . . . . . . . . . . . . . . . . . . . . . .  25

i

t_resume  . . . . . . . . . . . . . . . . . . . . . .  26

t_rname . . . . . . . . . . . . . . . . . . . . . . .  26

t_set_delay_ff  . . . . . . . . . . . . . . . . . . .  26

t_setmode . . . . . . . . . . . . . . . . . . . . . .  27

t_start . . . . . . . . . . . . . . . . . . . . . . .  27

t_stop  . . . . . . . . . . . . . . . . . . . . . . .  28

t_suspend . . . . . . . . . . . . . . . . . . . . . .  28

t_unhook_int  . . . . . . . . . . . . . . . . . . . .  28

TCHRT V3.00 Registration . . . . . . . . . . . . . . . .  30

ii

TCHRT User Guide

Introduction

TCHRT  (Turbo  C  High  Resolution  Timer) is a software library  that

provides the software developer with  reliable  timing  functions that

work across  the entire Intel 80xxx PC compatible hardware environment

with microsecond resolution. The TCHRT timing logic  calibrates itself

to the host PC clock rate and processor environment at run-time,  so a

single executable program will yield  accurate  results  regardless of

the host microprocessor and clock speed.

TCHRT  is  the  definitive choice for any PC application that requires

precision timing.  Some typical TCHRT applications are:

Source Code Timer/Profiler

By  adding timer calls to source code  under  development,  TCHRT  can

generate a detailed summary  of  execution  time by program subroutine

and immediately  illustrate  the  software's  "hot  spots"  that might

benefit from further code or algorithm optimization.  Unlike "binning"

profilers  that display the number of "hits" in  a  range  of  program

addresses, TCHRT provides  a  direct one to one correspondence between

source code and elapsed time with user defined granularity.

Hardware Performance Evaluator

The high resolution and  self  calibration  features  of TCHRT make it

ideal  for  insertion  in code to evaluate hardware performance.  Disk

drives,  numeric processors and CRT controllers  can  all  have  their

throughput measured very  accurately  with  TCHRT,  and  with numerous

timers  at  the  users   disposal,   many   device  functions  may  be

independently  measured  in  a single run.  The formatted timer report

generated  by  TCHRT  makes  compilation  and  interpretation  of test

results painless.

Process Control

With TCHRT, process control applications that require  precise timings

or  precision  delays  are  easy  to  implement.    No longer will the

application developer have to make an educated guess as to how  long a

critical interrupt is taking to service an I/O port as TCHRT  can time

virtually any hardware or software interrupt and provide the developer

with  precise  accurate  timings  of  the  critical  procedure.    The

precision delay  functions  allow  the  programmer  to  implement data

acquisition loops with microsecond resolution.

Timing and Scoring

TCHRT  is the perfect "timer engine" to build  a  timing  and  scoring

system for use in nearly any competitive event.  With  its microsecond

resolution and self calibration features, TCHRT surpasses conventional

sports   timing  equipment  resolution  and  allows  the   application

developer to  package  high  resolution  timing  and  complete scoring

functionality in a single package.

Library Source Code Included

Since the source code  to  the  TCHRT  linkable libraries is included,

users needing  a particular timing function not implemented can modify

the TCHRT functions to suit their particular requirements.

Functional Overview

The  functionality  of  TCHRT  can  be divided into  several  distinct

categories of timer services:

TCHRT Timer Logic

These functions initialize  the  TCHRT  timing  system,  calibrate the

timing logic, provide a low level interface to the PC timing hardware,

and perform miscellaneous timing functions that  are  generally hidden

from  the  user.  Some of these functions will be of interest to users

who  need to design their own timer functions and want to build on the

TCHRT low level timer logic.

Generic Timer Functions

These  functions allow the user to  control  arbitrary  event  timers.

Functions exist to start, stop,  suspend,  resume,  name,  get elapsed

time  and activation counts, and to  generate  detailed  reports  that

summarize  all  timer  activity.  Timer  reports may be output to  the

display, a disk  file,  or  to  the printer. A setmode function allows

users to run timers  with  interrupts enabled or disabled.  The number

of timers active concurrently is restricted  only by available memory.

BIOS Interrupt Timer Functions

These  functions  are  similar  to  the Generic Timer  Functions,  but

operate only on BIOS  interrupts.   Using these functions the user may

install  timer   logic  in  any  PC  interrupt  vector,  and  complete

information on  that interrupt's activity can be generated.  Interrupt

timer  reports  list  interrupt  activity  broken  down  by  interrupt

function (contents of the AH register when the  interrupt  is invoked)

and text  files  containing  interrupt  function  descriptions  may be

loaded to describe each  interrupt  function  timer  in the BIOS timer

report.    Interrupt  description  files  are  provided  for important

software interrupts in the PC/MSDOS environment.   Multiple interrupts

may be timed concurrently.

Precision Delay Functions

These functions allow the user  to  generate  delays  with microsecond

accuracy. Functions exist to  generate  delays,  query the delay logic

for available  delay resolution and minimum delay time possible (these

are hardware dependent), and to  "fine tune" the delay calibration for

one or more specific delay lengths.

TCHRT  2

Theory

TCHRT works by directly  interfacing  with channel 0 of the 8253 timer

chip found in all compatible PCs.  Channel 0 counts  from  65535 to 0,

when  it  triggers  BIOS  interrupt  8 and begins counting from  65535

again.  Interrupt 8 is the hardware timer interrupt, and it increments

the DOS time  of  day  data  word  in  the  BIOS data area, checks for

diskette motor activity  and  shuts  down  the  motor  if  needed, and

finally calls BIOS interrupt 1C (hex), which is known as  the software

timer tick.  At boot time, 1C contains an  IRET  instruction,  meaning

there is no routine installed in  that  vector.   Since the 8253 has a

period of 838 nanoseconds, the hardware timer tick occurs every 54.925

milliseconds, or 18.2 times a second, which is  the  standard software

time resolution of the PC.   The  8253  count can be read by accessing

the appropriate IO port, and by reading the 8253 count along  with the

low word DOS time of day count, we  can  construct  a  high resolution

time stamp, which  is  the  base  unit  of  time  for TCHRT.  The only

modifications we make to the PC  environment  is to change the mode of

channel 0 to give the output count a 50 percent duty cycle, which does

not  impact  any  other  PC  operations.    Thus  we  have  a  simple,

unobtrusive,  highly  accurate  time  reference  with  838  nanosecond

resolution.    Once  a highly accurate time reference is available, we

can use it to calibrate a  simple  function that contains an inner and

outer loop to generate precision delays.

The accuracy of TCHRT is as  accurate  as  the 8253 timer chip and the

ability of the software to measure the overhead in retrieving the 8253

and DOS time of day data.  With the advent of  advanced  chips  in the

80286 and 80386 class, the  effects  of  instruction  prefetch queues,

pipelining, and  cache memory become significant and the same sequence

of  instructions  may  take  different  amounts  of  time  to  execute

depending   on   the   interaction   of  the  processor's  performance

optimization  mechanisms.    The  end  result  is  twofold:    TCHRT's

calibration  routines may experience inaccuracies, and software  under

test  may  give  different timing results on different runs.  Normally

the "jitter" introduced by these conditions  is  on the order of a few

microseconds.

A  major  source  of  apparent  erratic timer behavior comes from  the

interaction of asynchronous software and hardware interrupts  that may

occur  during a timed interval.  While  the  PC/MSDOS  environment  is

essentially single-threaded,  there  is  enough  "background" activity

occurring  in the form of timer and  keyboard  interrupts  that  these

interrupts  may occur during an timed interval,  and  the  time  these

interrupts  take  to  complete  are  naturally  counted  in the  timed

interval.   For very short duration timing a  TCHRT  timer  mode  with

interrupts disabled is available,  but  that  may  not be suitable for

longer  duration  timings,  or  when  concurrent  timer   activity  is

required.  Be aware that few results in  the  PC  environment  will be

exactly  repeatable,  and  that some variance in the timing results is

unavoidable.

The high resolution  delay  function of TCHRT is particularly affected

by  background  interrupts  and  processor   performance  optimization

mechanisms.    The  initial delay calibration will yield delays with a

variance of from one to three percent of the requested delay interval.

TCHRT  3

Additional functions are available to optimize  the  delay calibration

to a specific delay interval, and using these functions can reduce the

delay  error  to  between  zero  and  one  percent,  and thus they are

recommended for critical applications.

TCHRT Function Descriptions

The following is  a  brief  synopsis  of the functions found in TCHRT.

This chapter is meant to give  the  user  a brief overview, and is not

meant to be a definitive reference.  It  is  essential  that  the user

review the  complete  function  reference  in  the  Function Reference

chapter before using any TCHRT function.  Some functions may  not work

as they would appear at first inspection, and in order to achieve high

performance, TCHRT has a minimum of internal error checking to prevent

such catastrophes as NULL pointer references.

TCHRT Timer Logic

Initialization functions

t_request           Requests the number of timers needed.

t_start             Initializes the TCHRT timer logic.

t_stop              Shuts down TCHRT and frees allocated memory.

Internal functions

t_get               Gets TCHRT time stamp with interrupts

enabled.

t_hires_entry       Gets TCHRT time stamp with interrupts

disabled.  8253 count is reset.

t_hires_exit        Gets TCHRT time stamp and restores interrupts

t_diff              Calculates elapsed time difference between

two TCHRT time stamps.

t_calib             Calibrates TCHRT timer logic, resets all

timers, and calls the delay calibration

function.

t_bios_entry        Receives timer start request from interrupt

service routine.

t_bios_exit         Receives timer stop request from interrupt

service routine.

t_bios_load_desc    Loads interrupt function description file

t_hook_int          Installs timer calls in specified interrupt

vector.

t_unhook_int        Removes timer calls from specified interrupt

vector.

t_delay_calib       Calibrates delay logic.

t_request is called by the user to specify the number of timers needed

for generic timer  operations  if  the  user  wishes  to  override the

default  number  of  timers provided (10).    Since  each  timer  data

structure is  allocated dynamically, the number of timers available is

restricted by the amount of heap available and the constraints  of the

memory model you are using. If  t_request  is used in your program, it

must be called before t_start.

TCHRT  4

t_start allocates the appropriate  amount  of  heap  for the number of

timers requested, initializes those  timers,  and  calibrates  the low

level timer routines.   t_start  must always be invoked prior to using

any other timer functions with the exception of t_request.

t_stop deallocates all allocated heap  associated  with  generic timer

activity and shuts down the timer logic.  t_stop must be  called prior

to program termination, and  no  further calls to TCHRT should be made

unless t_start is invoked first.

The TCHRT internal functions of primary interest to the programmer are

t_get, which is the interface to  the  8253 timer chip;  t_diff, which

calculates  the  elapsed  time  between  two  TCHRT  time stamps;  and

t_calib, which calibrates the  timer  and delay logic to the host run-

time environment.

Generic TCHRT Timer Functions

This class of functions implement general purpose timer functionality.

t_entry             Starts the specified timer.

t_exit              Stops the specified timer.

t_suspend           Suspends the specified timer.

t_resume            Resumes the specified suspended timer.

t_setmode           Specifies timer mode - interrupts on or off.

t_ask_timer         Gets accumulated activations and elapsed time

for the specified timer

t_cvt_time          Converts microseconds to MM:SS.xxxxxx string.

t_reset             Resets one or all timers.

t_report            Generates timer report

t_name              Associates string with specified timer for

display in timer report.

t_rname             Specifies title of timer report.

t_fname             Specifies destination file for timer report.

t_set_report        Specifies type of timer report generated.

t_entry  and  t_exit  provide  the  mechanism  to  start  and  stop  a

microsecond timer.  Timers are specified by number from 0 to n-1 in C,

and  1  to  n in Pascal.  The number of  timers  available  is  10  by

default, but the user can use  t_request  to request as many timers as

available heap allows.  Multiple timers may be active at the same time

if interrupts are enabled.

t_suspend and t_resume suspend and resume a timer previously activated

by t_entry.  Timer activation count is not  incremented  when t_resume

is called, which is useful if you are using timer activation counts to

track subroutine  activations in a profiling application, and for some

reason you wish to pause  the timer during the subroutine's activation

(perhaps before a call to another routine) and then restart it.

t_setmode  allows  the  user to specify whether  timers  operate  with

interrupts enabled or disabled.   With  interrupts  enabled, intervals

greater than 54925 microseconds may  be timed, and multiple timers may

be active concurrently.  With  interrupts  disabled,  the  timer logic

will "roll over" after 54925 usecs,  and  only one timer may be active

at any time.  Timing short duration events with interrupts disabled is

TCHRT  5

useful  if  background   system  interrupts  are  causing  significant

variations in timer results.

t_ask_timer will return to  the  user  the  activation count and total

elapsed time in microseconds for the specified timer.  t_cvt_time will

convert  a microsecond time count into a  printable  string  with  the

format MM:SS.xxxxxx.  Timer activation counts and  accumulated elapsed

time can be reset to zero by using t_reset.

t_report generates a complete  listing  of  all  timer activity.  This

listing can be directed to the display, a  disk  file,  or  a printer.

t_rname allows the user to give the timer report a  descriptive title.

t_fname overrides the default disk file name for  timer  reports going

to disk.  t_name allows the  user  to assign a descriptive text string

to  a  timer number - this string will be printed next  to  the  timer

number in the timer report.    Finally,  t_set_report  determines what

timers are listed on  the  timer report: either only those timers that

had  been activated, or all timers,  regardless  of  their  activation

count, up to the highest timer number activated.

BIOS Interrupt Timer Functions

This  class  of  functions   implement   the   BIOS   interrupt  timer

functionality.

t_bios_start        Installs timers in specified interrupt

vectors.

t_bios_stop         Removes all timers from interrupt vectors.

t_bios_suspend      Suspends specified interrupt timing.

t_bios_resume       Resumes interrupt timing in the specified

vectors.

t_bios_set_user     Assigns timer to user specified interrupt.

t_bios_set_file     Assigns interrupt function description file

name to the user specified interrupt timer.

t_bios_ask          Returns activations and elapsed for the

specified interrupt function

t_bios_reset        Resets one or all function timers of the

specified interrupt vector.

t_bios_report       Generates BIOS interrupt timer report.

t_bios_rname        Specifies title of BIOS timer report.

t_bios_fname        Specifies destination file for BIOS timer

report.

t_bios_start  initializes  BIOS interrupt timing  for  the  interrupts

specified.    Heap  is  allocated  for the required  data  structures,

interrupt function description files  are  loaded  if available, and a

new interrupt service  routine  for  each  interrupt  to  be  timed is

inserted in  the  interrupt  vector  table.    t_bios_stop essentially

reverses  this  process,  freeing  allocated  heap  and  restoring the

previous interrupt service vectors.    The  BIOS timer data structures

are separate from the generic timer structures, and thus t_request has

no effect on BIOS interrupt timer activity.

t_bios_suspend and  t_bios_resume allow the user to suspend and resume

BIOS interrupt timing  of  the specified interrupt(s).  All interrupts

to be timed must be installed by t_bios_start, but timing can  then be

immediately suspended by t_bios_suspend until the point in the program

TCHRT  6

that the user actually wants timing  to  begin, when it can be resumed

with t_bios_resume.

t_bios_set_user enables the user to specify an interrupt  to  be timed

that has not been provided for by TCHRT.  Each interrupt that is timed

requires new interrupt  service  routine that when activated, starts a

timer, calls the old vector, then stops the appropriate timer.   TCHRT

provides new timer ISRs for the major device, EMS,  Mouse,  and  MSDOS

interrupts, along with  a  spare  ISR  that  can be used for any other

interrupt the user might want to time.   t_bios_set_user  assigns this

spare timer ISR to the specified interrupt, allowing the user  to time

any interrupt vector in the PC environment.   If  the  interrupt  is a

software interrupt that has specific functions (as requested by the AH

register when the  interrupt  is  invoked)  the  user  may  specify an

interrupt function description  file using t_bios_set_file which loads

a text file containing interrupt function descriptions if that file is

in the current directory.    Interrupt  function description files are

provided for all the software interrupts TCHRT has standard timer ISRs

for.

t_bios_ask   and  t_bios_reset  function  like  their  generic   timer

counterparts and allow the user to request specific BIOS timer results

or to reset  those results to zero.

t_bios_report, t_bios_rname, and  t_bios_fname modify and generate the

BIOS timer report in the same way as their generic  timer counterparts

t_report, t_rname, and  t_fname  generate and modify the generic timer

report.

Be sure and carefully review the documentation for t_bios_start before

using the BIOS interrupt timing functions.

Delay Functions

This class of functions  implement  the  microsecond  resolution delay

functionality.

t_ask_delay         Requests parameters for requested delay time.

t_do_delay          Executes delay with interrupts disabled.

t_do_delay_wints    Executes delay with interrupts enabled.

t_calc_delay_ff     Optimizes delay calibration to a specified

delay duration.

t_get_delay_ff      Returns delay calibration optimization.

t_set_delay_ff      Sets current delay calibration optimization.

t_min_delay         Returns minimum delay possible with current

hardware.

t_res_delay         Returns delay resolution possible with

current hardware

t_ask_delay   requests  delay  parameters  for  the  specified   delay

interval.    These  parameters  are  stored in a user allocated  TCHRT

defined  data structure and are passed  to  either  t_do_delay,  which

generates  the  delay  with  interrupts disabled, or t_do_delay_wints,

which generates the delay with interrupts enabled.

t_calc_delay_ff,  t_set_delay_ff, and t_get_delay_ff, allows the  user

to  have  the  TCHRT  delay  logic  "fine  tune"  its calibration to a

TCHRT  7

particular delay interval.   t_calc_delay_ff  calculates  and installs

the  new  delay  calibration  optimization, t_set_delay_ff enables the

user  to   install   a   previously   calculated   optimization,   and

t_get_delay_ff retrieves the current delay optimization.

t_min_delay returns the minimum  delay  possible  in the host run-time

environment.  t_res_delay returns the resolution of the delay function

in the host run-time environment.

Examples

While  the  accompanying  pages  spell  out the functions of TCHRT  in

detail, as the saying goes, "a picture is worth a thousand words".  In

keeping with that philosophy,  we  have  included several source files

that demonstrate different  areas  of TCHRT functionality. They are as

follows:

DEMO             Basic  TCHRT operation.  Shows inline and  subroutine

timing, report generation.

TESTBIOS         BIOS interrupt timing.

SEEKTEST         Hard disk seek  test  benchmark.    Uses  both inline

timers and BIOS interrupt timing to  fully  test disk

seek performance.

DELAY            Demonstration of high resolution delay functionality.

WATCH            Full function Taylor Split stopwatch.   Shows  how to

activate timers with a user written interrupt service

routine.

These programs are carefully commented and  provide  valuable insights

into effective TCHRT operation.

Technical Support

Technical  support for licensed users of TCHRT  is  available  by  the

following methods:

1.  By US Mail directed to P.O. Box 22, Mt. Pleasant MI 48804.

2.  By   Compuserve  electronic   (Easyplex)   mail   directed   to

73047,1765.

Shareware

This software is distributed as "Shareware", which allows  you  to use

the complete product  on  an  evaluation  basis  to get a feel for its

quality, functionality, and value  to  you as a developer or end user.

At the end of your evaluation (14 days is a  widely  accepted time for

TCHRT  8

the evaluation period) you may continue to use the software product by

sending in a registration  fee  to  the  program author/vendor, or you

must discontinue use of the software and are free to pass your copy on

to another user for evaluation.   The Shareware concept allows quality

software  to   be  distributed  at  far  less  cost  than  traditional

commercial products, and allows the user to "try before you buy".

TCHRT is a single language functional subset of PCHRT,  the  PC  Timer

Toolbox distributed through traditional  commercial  channels  by Ryle

Design.   MSCHRT for Microsoft C and TPHRT for  Turbo  Pascal are also

available as Shareware, and  provide  the  same  suite of functions as

TCHRT.

Registration of TCHRT costs $20.00,  and in return users receive large

and small model object  libraries,  complete  library source code, and

technical support by US  Mail  and  Compuserv  Email.  Distribution of

TCHRT object code in commercial applications is not permitted.

Users  developing commerical applications or needing additional  timer

functionality should purchase PCHRT, the commerical version  of TCHRT,

which costs $49.95.  PCHRT includes all the functions of  TCHRT, along

with three  new timer functions, five new synchronous alarm functions,

four functions to run the timer tick interrupt at a user defined rate,

and two utilities to  time  execution  and  profile interrupts of most

executable programs without recompilation.  PCHRT comes with libraries

for  Turbo  C  and MSC in small, medium, compact, and large models, as

well  as units for Turbo Pascal V5 and V5.5.  Full library source code

for both Pascal and C is provided, along  with  a  printed  manual and

telephone technical support.  Distribution  of  PCHRT  object  code in

commercial applications is permitted without royalties.

An order form  for  these  products  appears  as the last page of this

document.

TCHRT  9

TCHRT  10

TCHRT Function Reference

t_ask_delay

Purpose          Fills delay parameter structure with  required values

for requested delay interval

C                void  t_ask_delay(long  unsigned  usec,  t_delay_type

*dp)

Synopsis         Parameters accepted by the delay functions are stored

in  a  TCHRT  defined,  user allocated data structure

tdelay_type.   t_ask_delay accepts the desired  delay

interval  in  microseconds,  and  fills   the  passed

tdelay_type structure with the appropriate parameters

to generate the requested delay.  This data structure

is    then    passed    to   either   t_do_delay   or

t_do_delay_wints,   where   the   actual   delay   is

generated.

Caveat           None

See Also         t_do_delay, t_do_delay_wints

t_ask_timer

Purpose          Returns activation  and  elapsed  time  count  for  a

specified timer

C                void t_ask_timer(int tnum, long unsigned  *hits, long

unsigned *elapsed)

Synopsis         The  user may retrieve activation counts and  elapsed

time  (in  microseconds)  for  the  specified  timer.

Timers are specified from 0 to n-1 in C and from 1 to

n in Pascal.  The number of timers n defaults  to 10,

but t_request  may  be  called  prior  to  t_start to

increase that number.

Caveat           No range  checking is done.  A NULL pointer reference

is possible if information on a non-existant timer is

requested.

See Also         t_request, t_reset

TCHRT  11

t_bios_ask

Purpose          Returns activation count and elapsed time for  one or

all functions associated with a BIOS interrupt timer

C                void  t_bios_ask(int  whichint, int  whichfunc,  long

unsigned *hits, long unsigned elapsed)

Synopsis         The users passes a single BIOS  timer  interrupt mask

(see  t_bios_start) and either an interrupt  function

in the range of 0 to 255 or -1, which causes  all the

interrupt   function  timers  to  be  summed.     The

accumulated number of activations and elapsed time in

microseconds is returned.

The summation of all function  timers  by  passing -1

feature is useful if a hardware  interrupt  that does

not have specific function  classes  is  being timed.

Interrupt functions are  specified by the contents of

the  AH  register,  and  since  the  AH  register may

contain  random, non-specific values when a  hardware

interrupt  is invoked, many timers for the  interrupt

will show some activity.   These  must  be  summed in

order to  determine  the  activity  of  the  hardware

interrupt.

Caveat           No range  checking is done.  A NULL pointer reference

is possible if information is  requested  on  an non-

existant interrupt timer.

See Also         t_bios_start, t_bios_reset

t_bios_entry

Purpose          Receives  timer  start  requests  from  an  interrupt

service routine

C                void t_bios_entry(int whichint, int whichfunc)

Synopsis         BIOS interrupt vectors  being timed call this routine

to start the appropriate  timer  for  this interrupt.

Parameter whichfunc is the value in  the  AH register

when   the  interrupt  was  invoked,  which  is   the

interrupt function requested in a software interrupt.

Caveat           This is  an internal TCHRT procedure and is not meant

to be called by the user.

See Also         t_bios_exit

TCHRT  12

t_bios_exit

Purpose          Receives  interrupt  timer  stop  requests   from  an

interrupt service routine

C                void t_bios_exit(int whichint)

Synopsis         BIOS interrupt vectors call this routine to  stop the

appropriate timer for this  interrupt.    The current

active function timer  for this interrupt is stopped,

and  if  the  interrupt  was entered recursively, the

previous function timer is restarted.

Caveat           This is  an internal TCHRT procedure and is not meant

to be called by the user.

See Also         t_bios_entry

t_bios_fname

Purpose          Specifies a name for the  disk  file  created  by the

BIOS timer report if the disk  destination  option is

selected

C                void t_bios_fname(char *fname)

Synopsis         By calling t_bios_report(1)  the BIOS timer report is

generated  and  sent  to  an  ASCII disk file.    The

default  name   for   this   file   is  BIOSTIME.TXT.

t_bios_fname  allows the user to specify a  different

filename,   with   complete   disk,  path,  and  file

specification.

Caveat           Maximum  disk/path/filename  string   length   is  67

characters

See Also         t_bios_report

t_bios_load_desc

Purpose          Loads   an   interrupt   description  file  into  the

appropriate interrupt timer data structure

C                void t_bios_load_desc(int whichint, char *data_file)

Synopsis         Each  interrupt  vector  being  timed   contains  256

timers,  one for each possible interrupt function  as

indicated  by  the  AH   register.      An  interrupt

description  file  contains  brief  twenty  character

descriptions  of  each  interrupt  function,  and  if

loaded, these will appear  along  with  the interrupt

function  number in the BIOS interrupt timer  report.

TCHRT  13

whichint   is  a  BIOS  timer  interrupt  mask   (see

t_bios_start),  and  data_file  is the disk/path/file

descriptor.  No error is generated  if  the specified

file is not found.

See  one of the supplied interrupt description  files

(such as 10.INT) for the layout of these files.

This routine is called automatically by t_bios_start.

Caveat           No range checking is done.  NULL pointer reference is

possible.

See Also         t_bios_start, t_bios_set_file

t_bios_report

Purpose          Generates BIOS interrupt timer report

C                void t_bios_report(int whereto)

Synopsis         This routine generates a  complete  timer  report for

all active BIOS  interrupt timers.  whereto specifies

the report's  destination - 0 sends the report to the

display, 1 to a disk file, and 2 to a printer.

Caveat           None.

See Also         t_bios_rname, t_bios_fname

t_bios_resume

Purpose          Resumes suspended BIOS interrupt timing

C                void t_bios_resume(int biosmask)

Synopsis         This  routine  enables   previously   suspended  BIOS

interrupt  timing.  biosmask contains the BIOS  timer

bitmask (see t_bios_start)  of the interrupts to have

timing re-enabled.

Caveat           No range checking is done.  NULL pointer reference is

possible if a interrupt is  referenced  that  was not

setup by t_bios_start.

See Also         t_bios_suspend, t_bios_start

t_bios_reset

Purpose          Resets one or  all  timers  associated  with  a timed

interrupt

TCHRT  14

C                void t_bios_reset(int whichint, int whichfunc)

Synopsis         This function allows the  user  to  reset  either one

specific  or  all  function  timers associated with a

timed interrupt.  Specific function timers are in the

range of 0 to 255.  Passing -1  for  whichfunc resets

all  256  timers  associated  with a timed interrupt.

The interrupt is specified  by  whichint  which  is a

single BIOS interrupt timer mask (see t_bios_start).

Caveat           No range checking is done.  NULL pointer reference is

possible   if  a  non-existant  interrupt  timer   is

specified.

See Also         t_bios_start, t_bios_ask

t_bios_rname

Purpose          Specifies title for BIOS timer report

C                void t_bios_rname(char *rname)

Synopsis         This function  allows the user to specify a title for

the  BIOS  interrupt  timer  report.    If   none  is

specified,  the  title  used  is Bios Interrupt Timer

Summary.  The title is horizontally centered.

Caveat           Max string length is 80 characters.

See Also         t_bios_report, t_bios_fname

t_bios_set_file

Purpose          Sets the interrupt  function description file for the

user specified interrupt timer

C                void t_bios_set_file(char *user_int_file)

Synopsis         When a user specifies the optional interrupt  to time

using  t_bios_set_user, a file of interrupt  function

descriptions   may  also  be  specified  using   this

function.

Caveat           Max file description length is 67 characters.

See Also         t_bios_load_desc, t_bios_set_file

t_bios_set_user

Purpose          Specifies the optional interrupt to time

TCHRT  15

C                void t_bios_set_user(int user_int)

Synopsis         The  TCHRT  BIOS timing functionality  has  built  in

support for the various BIOS interrupts of interest -

disk, CRT, serial,  parallel, DOS, EMS, keyboard, and

mouse.   This function allows the user to specify any

other interrupt to be timed, and when t_bios_start is

invoked with the  USER  bit set in the timer bitmask,

the interrupt specified by t_bios_set_user  will have

timers inserted in its vector.

Caveat           The  ISR  supplied  that  is  hooked  into  the  user

specified interrupt (t_new_user in the  .ASM modules)

does not propagate the  FLAGS  register.   If you are

attempting to time  a software interrupt that returns

meaningful information  in  the  FLAGS  register, you

should  modify  the  t_new_user  routine so that it's

last instruction is a RETF 2 instead of  IRET.   This

is documented in the t_new_user code.

See Also         t_bios_start, t_bios_set_file

t_bios_start

Purpose          Initializes BIOS interrupt timing

C                int t_bios_start(int biosmask)

Synopsis         This function initializes BIOS interrupt timing.  The

interrupts to be timed are  selected  by  passing the

appropriate bitmask:

CRT10       INT 10h Video Services

DISK        INT 13h Disk Services

COM         INT 14h Serial Port Services

KEYBD       INT 16h Keyboard Services

PRT         INT 17h Printer Services

DOS21       INT 21h MSDOS System Services

MOUSE       INT 33h Mouse Services

EMS         INT 67h EMS Services

USER        User Specified Interrupt

As an  example, to insert timers in the DOS interrupt

and   the   disk   interrupt,   one    would   invoke

t_bios_start(DISK+DOS21).

TCHRT  16

Each  interrupt timed  actually  has  256  individual

timers,  one  for  each  possible  value  in  the  AH

register,  which indicates what specific function  of

that interrupt has been requested.    The  timer data

structure   is   allocated  dynamically,   and   each

structure requires approximately 9200 bytes, so there

is a limit imposed by  the memory model being used as

to  how  many  interrupt  timers  may  be  requested.

t_bios_start returns FALSE  in  C  if  the  amount of

memory  needed  is not available,  while  a  run-time

error occurs in Turbo Pascal.

If the USER bitmask is  to  be  used, t_bios_set_user

must  be  invoked  prior  to t_bios_start to indicate

what interrupt the USER bitmask will time.

Interrupt   function   description  files   for   the

supported interrupts are supplied, and will be loaded

if they are present  in  the  run-time directory.  As

these  will  dramatically  improve  the   information

presented in the BIOS  interrupt  timer  report, they

are  recommended.  t_bios_set_file associates a  file

with the USER selected interrupt.

If testing code with  BIOS  interrupt  timing active,

and the code fails prior to calling t_bios_stop, then

you  should immediately reboot your computer, as  the

interrupt vectors being timed  are  pointing  to non-

existant code.

Caveat           Approximately  9200  bytes  are  required   for  each

interrupt to be timed.  Memory constraints can affect

this function.

See Also         t_bios_stop,     t_bios_set_user,    t_bios_set_file,

t_bios_suspend, t_bios_resume

t_bios_stop

Purpose          Stops all BIOS interrupt timing

C                void t_bios_stop(void)

Synopsis         This  function removes all timers from all  interrupt

vectors selected by t_bios_start and  deallocates all

memory associated with those timers.

Caveat           This function must be called prior to program exit in

order to restore the interrupt environment.    If the

code under  test  fails  prior  to  t_bios_start  the

interrupt  vectors  being  timed  will be pointing to

non-existant code, and a system hang is likely.

See Also         t_bios_start, t_bios_suspend, t_bios_resume

TCHRT  17

t_bios_suspend

Purpose          Suspends interrupt timing selected by t_bios_start

C                void t_bios_suspend(int biosmask)

Synopsis         BIOS  interrupt timing is initiated by  t_bios_start,

and terminated by t_bios_stop.   t_bios_suspend gives

the user the ability to selectively disable timing on

a particular interrupt or  interrupts  by  passing an

appropriate  biosmask  (see   t_bios_start).     This

suspended interrupt timing may  be  later  resumed by

invoking t_bios_resume.

Caveat           No range checking is done.  NULL pointer reference is

possible   if  a  non-existant  interrupt  timer   is

referenced.

See Also         t_bios_resume, t_bios_start, t_bios_stop

t_calc_delay_ff

Purpose          Optimizes  delay calibration  for  a  specific  delay

interval

C                float t_calc_delay_ff(long  unsigned  delay_time, int

ints_or_not)

Synopsis         This  function  optimizes  delay  calibration   to  a

specific  delay interval.  ints_or_not can be  either

INTS_ON,  indicating a delay with interrupts  enabled

should  be used for the optimization calculation,  or

NO_INTS_ON,   which  indicates  that  a  delay   with

interrupts disabled should be  used.    This function

sets a  global value that is used by t_ask_delay, and

the value is also returned,  so that it may be stored

for later use with the delay interval optimized.

Caveat           None

See Also         t_set_delay_ff, t_get_delay_ff, t_ask_delay

t_calib

Purpose          Calibrates the TCHRT timer and delay logic

C                void t_calib(void)

Synopsis         This function calibrates the  low  level  TCHRT timer

and delay logic.   It  is  called  by t_start, so the

user  need  not  explicitly  call this routine unless

additional delay calibration is needed after a system

TCHRT  18

parameter has been  modified  during a timing run - a

keyboard initiated system clock  speed  change  is an

example.    t_calib  calls t_delay_calib to calibrate

the TCHRT delay logic.

Caveat           None

See Also         t_delay_calib

t_cvt_time

Purpose          Converts a microsecond count to a MM:SS.xxxxxx format

string

C                char    *t_cvt_time(long    unsigned    usecs,   char

*time_string)

Synopsis         This function accepts a microsecond count and returns

a printable character string in a simple time format.

This function is useful to  the  user  who  wishes to

build custom timer reports.

Caveat           Destination string length should be 13 characters.

See Also         t_ask_timer, t_bios_ask

t_delay_calib

Purpose          Calibrates the TCHRT delay logic

C                void t_delay_calib(void)

Synopsis         This  function  calibrates the delay  loops  used  to

generate  precision  microsecond  delays.  Additional

calibration   is   available   from  t_calc_delay_ff.

t_delay_calib is called by t_calib, so the  user need

not call it explicitly.

Caveat           None

See Also         t_calib, t_calc_delay_ff

t_diff

Purpose          Calculates  the difference  between  two  TCHRT  time

stamps

C                long  unsigned  t_diff(TCHRT_type  *start, TCHRT_type

*stop)

TCHRT  19

Synopsis         This function returns the difference  in microseconds

between two TCHRT time stamps, which are generated by

calls  to  t_get.   t_diff and t_get  are  low  level

functions  that  are  of  interest primarily to users

building timer functions not implemented in TCHRT.

Caveat           Don't pass the time stamps in the wrong order.

See Also         t_get

t_do_delay

Purpose          Generates   a   precision  microsecond   delay   with

interrupts disabled

C                void t_do_delay(tdelay_type *delay_params)

Synopsis         This function generates a precision microsecond delay

with interrupts disabled.   Using  this  function for

delays greater than 54925 microseconds will cause the

PC  clock to lose time.   Delay  parameters  of  type

tdelay_type for  the  delay  interval  desired    are

generated by t_ask_delay.

Delay interval error will be in the range  of  one to

three percent if t_calc_delay_ff is not used, zero to

one percent if it is.

Caveat           Interrupts are disabled during the delay period

See Also         t_ask_delay, t_calc_delay_ff, t_do_delay_wints

t_do_delay_wints

Purpose          Generates   a   precision  microsecond   delay   with

interrupts enabled

C                void t_do_delay_wints(tdelay_type : *delay_params)

Synopsis         This function is  identical to t_do_delay except that

interrupts are  enabled  during  the  delay interval.

This  will  introduce some additional  error  in  the

delay produced, but is more appropriate for  longer (

> 54925  usec)  delays.    Delay  parameters  of type

tdelay_type for  the  delay  interval  desired    are

generated by t_ask_delay.

Delay interval error will be in the range  of  one to

three percent if t_calc_delay_ff is not used, zero to

one percent if it is.

Caveat           Interrupt  activity  may  cause  random errors in the

delay interval produced.

TCHRT  20

See Also         t_do_delay, t_ask_delay, t_calc_delay_ff

t_entry

Purpose          Starts the specified general purpose timer

C                void t_entry(int timernum)

Synopsis         This   function   starts   a  general  purpose  timer

counting.  Timers are specified from 0 to  n-1  in C,

and from 1  to  n  in  Pascal.  The default number of

timers  is  10,  but that can be  modified  prior  to

calling t_start  by  passing  the  number  of  timers

needed to t_request.

Interrupts  are  normally  enabled  during  a  timing

interval.    t_setmode  can   be   used   to  disable

interrupts during any or all timing intervals.

Caveat           No range checking is done.    NULL  pointer reference

possible if non-existant timer is started.

See Also         t_exit, t_setmode

t_exit

Purpose          Stops the specified general purpose timer

C                void t_exit(int timernum)

Synopsis         This function stops a general purpose timer counting.

Activation count and accumulated elapsed time for the

specified  timer  is  updated.   Timers are specified

from  0 to n-1 in C, and from 1 to n in Pascal.   The

default  number  of  timers is 10, but  that  can  be

modified  prior  to calling t_start  by  passing  the

number of timers needed to t_request.

Caveat           No range checking is done.    NULL  pointer reference

possible if non-existant timer is started.

See Also         t_entry

t_fname

Purpose          Changes name of timer report file

C                void t_fname(char *filename)

TCHRT  21

Synopsis         t_report can send its  output  to  a disk file, which

has the default name TIMER.TXT.  This function allows

the user to assign a different disk/path/file name to

this file.

Caveat           No  checking  is  done  to  determine  if   the  file

description is valid.  Maximum  string  length  is 67

characters.

See Also         t_rname, t_report

t_get

Purpose          Retrieves  a   high   resolution   time   stamp  with

interrupts enabled

C                void t_get(TCHRT_type *timestamp)

Synopsis         This function interrogates the  8253  timer  chip and

the low 16  bits  of  the  DOS time of day data words

fills  the passed TCHRT_type data structure with  the

microsecond  resolution  timestamp.    This   is  the

fundamental unit of time for TCHRT.  The elapsed time

between two TCHRT_type  time stamps may be determined

by t_diff.

Caveat           Interrupts are enabled when t_get returns.

See Also         t_hires_entry, t_hires_exit

t_get_delay_ff

Purpose          Retrieves the current delay calibration optimization

C                float t_get_delay_ff(void)

Synopsis         t_calc_delay_ff  calculates  and  installs  a   delay

calibration  optimization  for   a   specified  delay

interval length.  Using this  additional optimization

can  vastly  improve  the  TCHRT  delay  accuracy  in

critical  applications.    t_get_delay_ff returns the

current  delay  calibration optimization  in  effect.

The default value (i.e. - no additional optimization)

is 1.00.

Caveat           None

See Also         t_calc_delay_ff, t_set_delay_ff

TCHRT  22

t_hires_entry

Purpose          Initiates timing with interrupts disabled

C                void t_hires_entry(TCHRT_type *timestamp)

Synopsis         This   function,   like   t_get,  retrieves  a  TCHRT

timestamp from the timing  hardware.    Unlike t_get,

t_hires_entry resets the 8253 timer  chip  to  0, and

interrupts are disabled at exit.  This  facilitates a

timing mode (set by  t_setmode)  that  functions with

interrupts disabled.

Caveat           Since the 8253 is reset, only one timer may be active

at  any  one  time.  The timer will "roll over" after

54925  usec,  so  this  mode  is  suitable  for short

duration timing  only.  This is a low level function,

and is not meant to be called by the user.

See Also         t_hires_exit, t_setmode

t_hires_exit

Purpose          Concludes a timing interval with interrupts disabled

C                void t_hires_exit(TCHRT_type *timestamp)

Synopsis         This  is the companion function to t_hires_entry  and

completes a timing interval with interrupts disabled.

At exit interrupts are enabled.

Caveat           See t_hires_entry.

See Also         t_hires_entry, t_setmode

t_hook_int

Purpose          Installs timers in the specified interrupt vector

C                void t_hook_int(int int_no)

Synopsis         This  function  installs  timers  in   the  specified

interrupt  vector  to enable precise  timing  of  the

interrupt activity.

Caveat           This is a low  level  function and is not meant to be

called by  the user.  Users wishing to understand how

TCHRT interrupt timing is accomplished should inspect

the source code to this routine.

See Also         t_unhook_int

TCHRT  23

t_min_delay

Purpose          Returns   minimum   delay   possible   in    run-time

environment

C                long unsigned t_min_delay(void)

Synopsis         The minimum delay interval that can  be  generated by

t_do_delay  or t_do_delay_wints varies with the  run-

time hardware environment.  This function returns the

minimum  delay in microseconds that can be  generated

in the current environment.

Caveat           None

See Also         t_res_delay

t_name

Purpose          Associates a descriptive string with a generic timer

C                void t_name(int timerno, char *timername)

Synopsis         This  functions  assigns   a  string  of  maximum  20

characters to the specified generic timer number, and

this string  is  then  displayed  next  to  the timer

number in the timer report generated by t_report.

Caveat           Maximum string length is 20 characters.

See Also         None

t_report

Purpose          Generates a report of all generic timer activity

C                void t_report(int whereto)

Synopsis         This  function  generates   a  report  of  all  timer

activity that was generated using t_entry and t_exit.

Parameter whereto specifies the report's destination:

0 sends the report to the CRT display, 1 sends  it to

a  disk file, and 2 sends it to  the  printer.    The

report  may  have  a  title  (see  t_rname),  and the

destination  disk   file   (if   used)   defaults  to

TIMER.TXT, but this can modified (see t_fname).

Timer  reports  have   two  formats,  and  these  are

selected by t_set_report.   The HIGHWATER report type

shows  the  activity  of  all timers, including those

with zero activation counts, up to the highest number

TCHRT  24

timer activated.  The NONZERO report type  only shows

timers with nonzero activation counts.

Caveat           None

See Also         t_rname, t_fname, t_set_report

t_request

Purpose          Request specific number of generic timers

C                void t_request(int numtimer)

Synopsis         t_request  provides  a  mechanism  to   override  the

default   number  of  generic  timers  allocated   by

t_start.  If the user requires more (or less) than 10

timers,   the  number  required  may  be  passed   to

t_request, and t_start  will attempt to allocate this

number of timers.

Caveat           Call before t_start.

See Also         t_start

t_res_delay

Purpose          Retrieves delay resolution of run-time environment

C                long unsigned t_res_delay(void)

Synopsis         In slower hardware environments, the delay resolution

of t_do_delay  and  t_do_delay_wints  may  be several

microseconds.   This  function  returns  the run-time

delay resolution in units of tenths of a microsecond.

For example, if t_res_delay returns  a  value  of 44,

that means the delay  functions  can  generate delays

starting  at   their   minimum   delay   length  (see

t_min_delay)   and   up    in   increments   of   4.4

microseconds.

Caveat           None

See Also         t_min_delay

t_reset

Purpose          Resets one or all generic timers

C                void t_reset(int timernum)

TCHRT  25

Synopsis         This  function  zeros  the accumulated activation and

elapsed timer counts for  a  specified  timer  in the

range of 0  to  n-1 in C, or 1 to n in Pascal.  If -1

is passed, all generic timers are reset.

Caveat           No range checking is done.  Null pointer reference is

possible if a non-existant timer is specified.

See Also         t_ask_timer

t_resume

Purpose          Resumes a suspended generic timer

C                void t_resume(int tnum)

Synopsis         Functions t_suspend and t_resume  allow  the  user to

stop and restart an active timer without incrementing

its activation count.

Caveat           No range checking is done.  NULL pointer reference is

possible if a non-existant timer is specified.

See Also         t_suspend

t_rname

Purpose          Assigns a title to the timer report

C                void t_rname(char *report_title)

Synopsis         The  generic  timer  report  generated   by  t_report

contains   a   horizontally   centered  title,  which

defaults to Timer Summary.  This function  allows the

user to specify an alternate title.

Caveat           Max string length is 80 characters.

See Also         t_report, t_fname

t_set_delay_ff

Purpose          Sets delay calibration optimization

C                void t_set_delay_ff(float delay_ff)

Synopsis         This function allows the  user  to  set the new delay

calibration  optimization  for  use  by  t_ask_delay.

Delay  calibration optimization  for  a  given  delay

interval  can be automatically calculated and set  by

t_calc_delay_ff.  Calling t_set_delay_ff with a value

TCHRT  26

of  1.00  removes  any delay calibration optimization

that was in effect.

Caveat           None

See Also         t_calc_delay_ff, t_get_delay_ff, t_ask_delay

t_setmode

Purpose          Sets generic timing mode

C                void t_setmode(int modetype)

Synopsis         Two modes of generic  timing  are  available: INTS_ON

enables generic timing  using t_entry and t_exit with

interrupts  enabled,  and  this  is the default mode.

NO_INTS_ON enables timing  with  interrupts disabled,

and this mode  is particularly suitable for extremely

short  duration  timing  that   may   be   skewed  by

background interrupt activity.   In  this  mode, only

one timer may be active at any time,  and  each timer

will   "roll   over"   after  54925  microseconds  of

continuous activation.

As long as only one timer is active at once,  the two

timer modes may be freely  mixed.  In order to change

modes, call t_setmode with the desired mode  prior to

calling the t_entry/t_exit functions.

Caveat           NO_INTS_ON  mode  is  not  compatible  with  multiple

active timers or BIOS interrupt timing.

See Also         None

t_start

Purpose          Initializes TCHRT timing functions

C                int  t_start(void)

Synopsis         This function  initializes  the  entire  TCHRT timing

system, and must  be  called prior to any other TCHRT

functions with the  exception  of  t_request.    If a

different  number  of  timers  other than the default

number provided (10) is required, t_request should be

called prior to t_start to specify the number needed.

In C,  if t_start can allocate enough heap to fulfill

the timer  request,  it  will  do  so,  calibrate the

timing  system,  and  return  TRUE.    If  the memory

request  fails,  it  will  return  FALSE.    In Turbo

Pascal, memory allocation failure  will  result  in a

run-time error.  Each timer requires approximately 35

bytes of heap.

TCHRT  27

The 8253 timer chip is set to mode 2.

Caveat           None

See Also         t_request, t_stop

t_stop

Purpose          Shuts down TCHRT timing logic

C                void t_stop(void)

Synopsis         This  function  deallocates  all heap associated with

generic timers and restores the  8253  timer  chip to

mode 3.  It must be called prior to program exit, and

no TCHRT calls (except t_request) are valid without a

call to t_start to reinitialize the timing system.

Caveat           None

See Also         t_start, t_request

t_suspend

Purpose          Suspends an active generic timer

C                void t_suspend(int tnum)

Synopsis         Functions t_suspend and t_resume  allow  the  user to

stop and restart an active timer without incrementing

its activation count.

Caveat           No range checking is done.  NULL pointer reference is

possible if a non-existant timer is specified.

See Also         t_resume

t_unhook_int

Purpose          Removes timers from the specified interrupt vector

C                void t_unhook_int(int int_no)

Synopsis         This  function  removes  timers  from  the  specified

interrupt  vector  to  disable  precise timing of the

interrupt activity.

Caveat           This is a low  level  function and is not meant to be

called by  the user.  Users wishing to understand how

TCHRT interrupt timing is  accomplished  may  want to

inspect the source code to this routine.

TCHRT  28

See Also         t_hook_int

TCHRT  29

TCHRT V3.00 Registration

Name ______________________________ Company ______________________________

Address __________________________________________________________________

City/State/Zip ___________________________________________________________

Telephone _______________________ Got TCHRT from _________________________

Version 3 Single Language Timer Toolboxes.  Includes large & small

model libraries (C), full library source, support by US Mail or

Compuserve Email.  License does not permit commercial distribution

of object code.

Qty _____ TCHRT V3 for Turbo C @ $20.00 each ..................... _______

Qty _____ MSCHRT V3 for Microsoft C @ $20.00 each ................ _______

Qty _____ TPHRT V3 for Turbo Pascal 5 & 5.5 @ $20.00 each ........ _______

Version 4 Timer  Toolbox. Supports  TC & MSC in small, medium,

compact, and large models, Turbo Pascal 5 & 5.5.  20 additional

functions to manage timers, profile interrupts, generate synchronous

alarms, run the timer tick interrupt at a user specified rate.

Two standalone utilities time execution and profile interrupts of

most executable programs without recompilation.  Full library

source in C & Pascal included.  Printed manual.  Support by

telephone, US Mail or Compuserv Email.  License  permits

commercial distribution of object code.

Qty ______ PCHRT V4.00 @ $49.95 each ............................. _______

Shipping in USA @ $2.00 .......................................... _______

Shipping outside USA @ $4.00 ..................................... _______

Subtotal ......................................................... _______

Michigan residents add 4% ........................................ _______

Total enclosed ................................................... _______

Payment method:  ___Check  ___Money Order  ___VISA  ___MasterCard

Card # ____________________________________________ Exp _________

Signature _________________________________________

Remit to Ryle Design, P.O. Box 22, Mt. Pleasant, MI 48804

Credit  card  customers  may  order  by  Email  on Compuserve  to

73047,1765,  or  by telephone at (517) 773-0587.  This is a voice

mail system.  Leave your name and number in voice mailbox #2, and

your call will be returned and your order taken.

Thank you!

TCHRT  30