Command-line diagnostic CHECKSYS.EXE

Dan Shearer, Computer Centre, University of South Australia

Comments and bug reports to:  ccdps@lux.levels.unisa.edu.au

I'd appreciate ideas or code for more checks.

Version 1.6 released to the public domain 22nd July, 1992.

Overview

---------

CHECKSYS is a command-line diagnostic program which either gives a short

description of a whole PC system, or returns a one-line description and

gives an errorlevel when asked about one aspect of the system.

The primary intention is for it to be used to determine system configuration

and driver status from batch files. It can also be handy when wanting a

quick idea of what a machine has during a system debugging session

without starting up a complicated package and poking around menus.

Reasoning Behind CHECKSYS

-------------------------

We have a very mixed hardware environment at UniSA, but like to have

generic batch files on the LAN. In our batch file that starts up Windows,

for instance, we have several CHECKSYS calls to determine what mode

Windows should be run in and what hardware drivers are needed, and also

to check and see if Windows is already running.

When to Use CHECKSYS

--------------------

Some things can be detected quite reliably from DOS using an IF EXIST...

test on a driver name. For example, if you have an Etherlink II network

card installed and you know its driver is called ELNKII$ then you can

say something like:

IF EXIST ELNKII$ ECHO   There is a 3Com Etherlink II card installed

If you don't know what the driver is called, a simple way to find out is

to use DOS 5's MEM /D command, which lists the names of all drivers loaded.

You can also use this method to determine the existence of extended memory,

expanded memory, and so on. It is a very crude check, no staus info etc but

works quite well. CHECKSYS is for when you need more detail; for

instance, EMM386 and HIMEM might be installed but the HMA not be

available.

How it Works and Acknowledgements

---------------------------------

It is a collection of routines that I have put together in Turbo Pascal

6.0, with assembler routines to check for CPU and FPU. All the detection

calls are in a self-contained TP unit, and I would be willing to release

this to the public domain as well if there was a call for it.

The commandline switch checking code in CHECKSYS is with the kind permission

of Rocky Stone, also of UniSA. Gary Oxton (once a UniSA student) wrote

the basis of the CPU-detection assembler I have used for his final-year

Computer Studies project. I have learnt about checking for memory types

from the InfoPlus source code, by Andrew Rossman and Steve Grant, and

used their LongCall assembler function, which overcomes a shonky nonfeature

of Turbo Pascal 6.0.

Ralf Brown's interrupt list is, of course, absolutely essential.

Syntax

------

CHECKSYS Vers 1.6                                          (C) 1992 Eyesright

Comments to Dan Shearer, University of South Australia

Dan.Shearer@UniSA.edu.au

CHECKSYS [/switches]         where switches for this free software are:

/ALL  Displays messages for all tests and returns errorlevel 0

/CPU  returns errorlevel 0-8 in order for:

8088, 8086, V20, V30, 80188, 80186, Unknown, 80286, 80386

/EMM  returns errorlevel 1 if EMM driver installed

/FTP  returns 1 if packet driver responding

/KBD  returns errorlevel 1 if extended keyboard installed

/MOUSE  returns number of mouse buttons (=0 for no mouse)

/NETBIOS  returns errorlevel 1 if NetBIOS responding

/PAR  returns number of parallel ports recognised by BIOS

/PROTMAN  1 if Microsoft Network Driver Interface (PROTMAN$) responding

/REMBOOT returns 1 if drive 0 parkable (ie not a network device)

/SER  returns number of serial ports recognised by BIOS

/VID  returns 1-10 in order for:

CGA, MCGA, EGA, EGA64, EGAMon, 8514, HercMon, ATT400, VGA, PC3270

/WIN  returns 0-3 in order for:

Win 3.x (/3), Win 3.x (/R or /S), Win/386 2.x, No Windows

/XMS  returns 2 for XMS & HMA avail, 1 for XMS but no HMA, 0 for no XMS

/87   returns 0-3 in order for: 80387, 80287, 8087, None

Each test also displays a detailed message to standard output.

Switches can be abbreviated to 1 letter. Errorlevel 99 for syntax error.

Examples:

CHECKSYS             displays the above help message

CHECKSYS /ALL        displays detailed results of all tests

CHECKSYS /MOUSE      displays detailed info about the mouse, and

returns errorlevels 0 for not present, 2 for two

button mouse present or 3 for three button mouse.

Remember when checking errorlevels in batch files that if a program

returns errorlevel n DOS also returns all other errorlevels from n-1 down

to 0. This is why (for instance) CHECKSYS /WINDOWS returns errorlevel 3 for

no windows and 0 for version 3.x. If it was 0 for no windows, you would

still have to check to see that it wasn't one of the others.

Bugs

----

BIOS is often wrong about printer and comms ports. Could add some better

checks.

The FPU check gives a silly result when used on a NEC V20 chip with an

Intel 8087 installed

Doesn't test for Weitek FPU

Doesn't distinguish OS/2 dos-boxes from Windows dos-boxes

Doesn't test for various network cards

Doesn't test for game port

Doesn't give video chipset particulars (might be useful for running

Windows with specific screen drivers)

Doesn't detect sound cards

Could possibly have a TSR check, ie syntax like:

CHECKSYS <unique TSR signature> [<optional range of interrupts>]