PMRADIO V1.1

by

Paul Ratcliffe

(c) Aug. 1997

This program is designed for use with the Reveal RA300 FM Radio Card. Its

operation is NOT supported on any other card, although I have received a

report that the RadioTrack card manufactured by AIMS Labs. also works.

The program is freeware and as such:

USE OF THIS PROGRAM IS ENTIRELY AT YOUR OWN RISK.

INSTALLATION

Copy the device driver FMRADIO.SYS and applications PMRADIO.EXE and

RADIO.EXE to a directory of your choice.

The OS/2 CONFIG.SYS file must be modified to load the device driver by

inserting a line like the following:

DEVICE=x:\path\FMRADIO.SYS [/p:port] [/v]

where x:\path is the drive and directory in which you installed FMRADIO.SYS

port    is the RA300 port address (in hex.).

The port addresses supported are 30F and 20F. The /P parameter is optional

and if not supplied, will default to address 30F.

The port address must match that which you have configured on the card

itself using jumper JP1.

The device driver is Resource Manager compliant and will not install if

some other device has already claimed the ports (the hardware actually

consumes 4 ports: 30C-30F or 20C-20F).

The /V parameter is also optional. It may be specified if you don't wish

the device driver to turn the radio volume down when shutting down OS/2.

You must reboot (in order to load the device driver) before you can use the

applications.

If you are upgrading from PM Radio 1.02 or earlier, you should run

UPGRADE.CMD in the directory containing your PM Radio INI file.

OPERATION

PM Radio is, as its name implies, a Presentation Manager application. It

can be started from the Workplace Shell, an OS/2 text mode session or even

directly from a DOS session.

The program is designed to operate much like a normal car radio, with

up/down tuning, scanning and volume controls, and preset memories which can

be activated at the touch of a button.

It is possible to drive the program entirely from the keyboard if the user

prefers - it is not necessary to have a mouse or other pointing device.

There is support for drag and drop colour and font changes, and the program

will maintain this information for the various states of controls which use

more than one colour/font.

TUNING

The frequency can be altered in steps of 100kHz (coarse tuning) or 25kHz

(fine tuning) between 87.5 MHz and 108.0 MHz using the tuning buttons.

Alternatively, the Frequency menu item can be selected which presents a

dialog box where you can enter a frequency directly or use the spin

buttons. Right clicking on the frequency window itself also brings up the

dialog box.

The colour of the text in the frequency window changes colour to indicate

whether a station is tuned correctly or not. The default colours are red

indicating off tune, changing to cyan when tuned correctly.

VOLUME

The volume control has 16 steps as that is all that the hardware supports.

A Mute function is also available, as sound is still audible at the minimum

volume level. Changing the volume automatically unmutes the sound.

The Dim function reduces the volume by a user configurable number of volume

steps. It is possible to change the volume level without affecting the Dim

state.

MEMORIES

The radio has 20 memories which may be selected at a touch of a button. The

default colour for the button text is black, changing to blue when the

station programmed in to that button is tuned in. Unallocated memories are

grey.

Each memory contains the frequency and volume assigned to a station and

optionally a name.

To store, edit and clear memory information, the Memory menu item is

selected. This brings up the dialog using the currently selected memory.

Right clicking on a preset button also brings up the dialog using the

details for the button clicked on.

The Load Current button loads the details for the currently tuned station

into the selected memory.

Undo restores the settings for this memory to what they were before the

current editing operation.

Clear is used to erase a memory.

Multiple memories may be changed at once by changing the Memory field.

Pressing Save saves all changes - Cancel aborts all changes.

OPTIONS

There are several which should be fairly self-explanatory.

Turning Full controls off removes the tuning and volume controls just

leaving the memory buttons. Keyboard control is always available even when

the controls are not displayed.

Confirm on Exit displays a confirmation box before the program is closed.

This allows you to exit with the radio on or off or to cancel the operation.

Mute whilst Tuning removes the pops and splats which can occur whilst

tuning the radio.

Dim level allows the user to configure the number of volume steps by which

the volume is reduced when the Dim command is executed.

Events active enables the events if they have been configured.

EVENTS

Events can be set up at a specified time of day (and optionally on every

hour or half hour thereafter) to Mute, Un-Mute, Dim or Un-Dim the radio.

This may be useful e.g. to leave the radio on in the background dimmed and

be brought up to full level on the hour for the news.

Up to 1440 different events can be configured and can be individually

enabled or disabled.

COMMAND LINE OPTIONS

There are four options available:

PMRADIO /m	Forces the program to start in the minimized state.

PMRADIO /r	Forces the program to start in the restored state.

PMRADIO /p	Resets the fonts and colours to their defaults.

PMRADIO nn	Starts the program and selects memory nn.

MISCELLANEOUS

An INI file is created in the same directory as the PMRADIO.EXE file to

hold all the settings required by the program. Nothing is written to the

OS/2 INI files.

Sometimes, it is possible for the hardware volume setting to get slightly

out of step with the software setting. The key Ctrl-Alt-Shift-R can be used

to reset this - the volume is reduced to minimum and then muted.

RADIO.EXE is a very simple command line driven program which you can use to

tune the radio, turn it on and off, and change the volume.

HISTORY

Version 1.1  10-8-97

Modified to allow tuning down to 87.5 MHz.

Multiple events are now possible instead of just 1.

Extensive support for drag and drop colour and font changes.

Add command line option to clear stored colour and fonts and return to

the defaults.

Device driver modified so that muting of radio on shutdown is optional.

Included simple command line program RADIO.EXE .

Version 1.02  12-7-97

Fixed a cosmetic problem - when muting or unmuting, the Mute button was

being redrawn before the icon was changed, causing a flash.

Added the option to disable muting during tuning.

German translation now available.

Version 1.01  26-6-97

Fixed an intermittent trap D when selecting the Memory menu item.

Fixed a problem with the resources - on some systems, the text of strings

was truncated.

Extracted the remaining text strings from the code to the resources in

preparation for a German translation.

Version 1.0  19-5-97

Initial release.

CREDITS

Thanks to Petric Frank <P.Frank@stgl.sel.alcatel.de> for the German

translation and to Dave Liquorice <allsorts@nexus.demon.co.uk> for Beta

testing and suggestions.

OTHER

If you use this program could you please let me know. Also if you have any

comments, criticisms, bug reports or anything that could be improved.

I will reply to all messages received.

Paul Ratcliffe:	Internet: paul.ratcliffe@virgin.net

Internet: paul.ratcliffe@bbc.co.uk

Netmail to 050/074 GT Power Network