Fm: 	ESS Marketing							Date: 5/29/98

To: 	Maestro-2E Notebook Customers

Subj:	Power Consumption Data on the Maestro-2E & AC97 Codec ES1920

NOTE:

The following data are measured with the Maestro-2E ES1978S rev. B chip and ES1920 AC97 codec. Please check with ESS FAEs if there is any question.

Maestro-2E ES1978S:

| Operating Mode | Typical IDDD at 3.3V | Power consumption: (IDDD X 3.3V) Watt |
| --- | --- | --- |
| D0: Fully on | 48.80 mA | 0.161 Watt |
| D1 (*): ASSP clock stopped | 34.84 mA | 0.115 Watt |
| D2 (*) Every module is clock-stopped except the PCI interface module & 49.152Mhz. | 7.85 mA | 0.026 Watt |
| D3hot-1 (**) PCI clock is stopped. 49.152MHz clock is active. | 5.05 mA | 0.016 Watt |
| D3hot-2 PCI clock is stopped. 49.152MHz is stoppped. | 0.40 mA | 0.001 Watt |

NOTE:

(*) D1 and D2 are programmable. M-2E provides registers 54h-57h to individually stop the clock of each module to provide greater granularity in ACPI power management.  Among them, ASSP (13.96mA) and WaveProcessor (25.79mA) consumes the most power.  Other noteable modules are SoundBlaster and 24MHz clock.

(**) In D3hot-1 mode, the 49.152Mhz clock input remains active. To save more power, the 49.152Mhz clock can be stopped by 1) putting the PCI bus into B3 state; 2) putting a transistor to stop the clock during D3hot-2. The transistor can be controlled via a GPIO pin from the system logic.

ES1920 AC97 codec:

| Operating Mode | Typical IDDD at 3.3V | Typical IDDA at 5.0V | D+A combined (IDDD x 3.3v +IDDA x 5.0V) |
| --- | --- | --- | --- |
| D0: Fully On | 9.0mA | 60.0mA | 0.329 Watt |
| D1: PR0 + PR1 + PR2 | 2.5mA | 11.0mA | 0.063 Watt |
| D2: PR0 + PR1 + PR2 + PR3 + PR4 | 1.0mA | 0.0mA | 0.003 Watt |
| D3hot: PR0 + PR1 + PR2 + PR3 + PR5 (no PR4) | 0.1mA | 0.0mA | 0.000 Watt |