UMB_GWJ.SYS - An Upper Memory Manager for MS-DOS

Version 1.0

UMB_GWJ.SYS allows you to register available memory above the 640KB boundary

but below the 1MB start of extended memory.  Once this memory is registered

by UMM.SYS, it becomes available for use by MS-DOS applications, or by

MS-DOS itself.

Memory above the 640KB base memory boundary but below the High Memory Area

(HMA) and extended memory is called Upper Memory, and is grouped into one

or more Upper Memory Blocks (UMBs).

UMB can be provided on 386+ by memory managers such as EMM386.EXE

or QEMM386.SYS, which switch the processor into virtual 8086 mode and use

mapping capabilities of processor itself. This approach has this

advantage, that it is independent from the mainboard design, but also has

some disadvantages:

1) Switching into v8086 mode slows down your computer.

2) You cannot use td386.exe and tdh386.sys.

3) There are some programs which refuse to work in protected mode.

The approach of using available memory (usually SHADOW RAM) has the following

advantages:

1) Uses memory which normally would be left unused.

2) Does not allocate any memory (or maybe about 80 bytes on some

computers).

3) TD386.EXE can be used.

4) You are able to load TD386.SYS and HIMEM.SYS high.

This version is limited to HT-320 chipset, but if you know how to enable

SHADOW RAM for writing in your computer, you can easily adapt it.

Installation.

Driver UMB_GWJ.SYS is installed in CONFIG.SYS. It must be run before

HIMEM.SYS and every program you want to place in UMB. The program

parameters specify UMB areas which you want to make available, e.g.

UMB_GWJ.SYS c800-F800 FB00-FC00

means that regions from C800:0 inclusive to F800:0 exclusive

and FB00:0 inclusive to FC00:0 exclusive will be available.

In CONFIG.SYS you also must place the line

DOS = UMB

or

DOS = HIGH,UMB.

You must enable SHADOW RAM for the areas you wish to use or

disable memory remapping in CMOS SETUP. This is essential. If you do not

do so, you may observe that the same memory is visible under two addresses

- at the end of XMS and in UMB. This may lead to some unpredictable

behavior of your computer.

It may be also necessary to place an EMMexclude line in SYSTEM.INI

for the regions you are enabling if 386ENHANCED mode of Windows 3.1

is used.

This is the exemplary CONFIG.SYS contents:

DOS = HIGH, UMB

DEVICE = C:\UTIL\UMB_GWJ.SYS C800-F800

REM we load XMS manager to UMB

DEVICEHIGH = C:\UTIL\HIMEM.SYS

REM other programs we wish to install in UMB

DEVICEHIGH = ...

INSTALLHIGH = ...

Note, that in MS-DOS 6.x you can use "/L:" parameters.

How does it work.

1) UMB_GWJ is run during system initialization when CONFIG.SYS is

interpreted. It must be the first XMS server.

2) Program enables SHADOW RAM for writing and copies part of its code into

the first specified UMB. If this area is too small, error message is

written to the console and program exits. Otherwise 2Fh interrupt vector

is intercepted by the part resident in UMB. Part of the program resident

in low memory is removed by DOS.

3) After installation of every driver DOS checks, if XMS driver providing

UMB is installed. Resident portion of UMB_GWJ.SYS will return the positive

result of this test.

4) After returning all the block addresses driver releases interrupt 2F

and returns the information, that there are no more UMB available.

5) At this moment driver is no more needed, and because it hasn't

allocated any memory block, it will be overwritten by the first program

installed in UMB.

Notes:

1) When you define DEBUG during compilation, you may observe how the driver

is working.

2) At the beginning of the driver some space is reserved for DOS memory block

header.

3) Driver tries to detect lack of DOS=UMB line in CONFIG.SYS.

Unfortunately, it works only in DOS 5.0 due to the minor change in XMS

detection algorithm in newer versions of DOS.

4) EMM386.EXE can be used together with UMB_GWJ.SYS. It allows for

enabling and disabling EMS what is not possible, when EMM386 provides UMB.

If you load EMM386.EXE to UMB you must ensure, that EMS page frame is

above the EMM386.EXE program area. On my computer I found out, that the

first half of ROM-BIOS is unused, so I placed page frame at E800. It was

not possible explicitely, but I accomplished it using I and X options -

I have excluded areas used by UMB_GWJ and included E800-E7FF area.

Compilation:

TASM UMB_GWJ.ASM

TLINK /t UMB_GWJ, UMB_GWJ.SYS

Notes on porting to other chipsets.

1) You must change the SHADOW RAM enabling procedure. You may also use

external driver to do so loaded before UMB_GWJ.SYS.

2) On some computers unused SHADOW RAM area is remapped to extended

memory. Enabling it causes, that the same area of memory is visible at two

addresses or that the access to the final part of extended memory is

impossible. You must inform all the programs about it. It can be done by

intercepting 15h interrupt function regarding extended memory size and

modify word field at 45H of undocumented LL structure (pointer to this

structure is returned in ES:BX by int 21h function 52h). Exemplary

program to be loaded immediately after UMB_GWJ is included in this

package.

Notes on DMA usage.

On some computers DMA access to UMB obtained by SHADOW RAM enabling is not

possible. Even if it is possible in some cases errors may occur. It is

related to ISA bus construction. If in the same 128KB area that UMB blocks

there is 16 bit I\O device memory, it can happen, that 8 bit DMA transfer

will be treated by such a device as an access to its memory and I/O cycle

will be switched into 16 bit mode. So happens, if error occurs during

reading of unbuffered A0-A16 address lines. It causes, that every other

transferred byte is 255. Such errors may occur in areas A000-BFFF,

C000-DFFF and E000-FFFF.

There is video memory in first area and it is usually not used for UMB.

In second area Video BIOS is mapped. We may switch it into 8-bit mode

or disable it. I know how to do it for Trident cards. When you disable

BIOS it may be necessary to always use hard reboot instead of soft one.

Third area contains system BIOS. Because it is not located on the

extension card, it shouldn't cause problems.

WARNING!!

DMA errors or lack of DMA access may cause erroneous data writes to the

diskette if:

- SMARTDRV is used for diskette caching and it is located in UMB.

- DOS Transfer Buffer is moved to UMB

How to contact author.

Send a mail to Internet address

gwj@j-23.p.lodz.pl

Acknowledgements:

I would like to thank Kenneth Gober for his upper memory manager. It has given

me the stimulus to interest in memory optimizing.