Here is some piece of software I wrote about two years ago. Yesterday, someone

suggested me to post it to comp.binaries.ibm.pc, and so I herewith give it to

the public. It's a bios driver for herkules graphics mode, which hooks into

INT 10h (video) and supports a new video mode #8 with 720x360 dots. As it uses

characters of 8x8, this means 90x45 characters for those (few) programs that

use bios for screen access. Any others might get somehow confused! Also, on

my machine to clear the WHOLE screen on MsDos CLS command, I had to remove

ANSI.SYS.

This program uses two different methods of scrolling. First, the usual one:

copy almost 32kB of waitstatey video memory. This is *SLOW*, but some programs

require it, so I made it default. Second: Change the CRT controllers display

start adress. This is *FAST*, but... Use option /F when starting this program

to activate this mode.

The source code (assembly) is included. I did it in MASM 4. To build it your-

self, use:

MASM HERKULES;

LINK HERKULES;

REM ignore warning NO STACK

DEL HERKULES.OBJ

EXE2BIN HERKULES.EXE HERKULES.COM

DEL HERKULES.EXE

The comments in the source are german :-)

I did quite some effort on using fastscrolling whenever it might be of

advantage. For example, if /F is given and the bios is requested to scroll

let's say a rectangular portion in the middle of the display. Then I will

change the display start adress, thus scrolling the WHOLE screen up, and then

scroll/rotate those areas which weren't supposed to be scrolled (those

surrounding the above rectangular) back using memory copying. Quite smart, eh?

But it was of no use. I found NO program that scrolls only PORTIONS of the

screen AND that works with my program. Sigh.

Something else: I stole the 8x8 characterset from the GRAFTABL command of DOS

and then included it into this program. This implies two disadvantages:

I might get sacked by uSoft and it uses 2048 bytes both on disk and in memory.

Hey, I just remembered one other feature: line printer support!

This program supports text AND graphic hardcopies on various printers.

There is a patch area at the beginning of the COM file containing pascal

strings (first byte containing length). Do modify it, use DEBUG or SYMDEB.

Then at offset 10Fh there will be a byte containing the number of the printer

to use for hardcopies. 0 means LPT1, 1 = LPT2 etc. Cereal printers :-)

(RS232) are not supported :-(

At offset 110h there is a pascal string which is sent BEFORE a text hard-

copy. This could be a form feed or I don't know what.

120h   pascal string sent AFTER a text hardcopy

130h   pascal string sent BEFORE a graphic hardcopy

140h   pascal string sent AFTER a graphic hardcopy

150h   pascal string sent BEFORE each line of graphics hardcopy

This should set the printer into some graphics mode and also

tell him how much data there will come, that is 720 bytes.

Each byte will contain eight vertical dots.

160h   pascal string sent AFTER each line of graphics hardcopy

This should include the command to move up the paper by

the appropriate amount required in graphics mode (8/72" ?)

Have fun

Stressi (q81955@pbhrzx)