# Solo-1 with EEPROM interface update 2/19/98

For sound cards designs:

To add EEPROM interface onto the existing Solo-1 (revision D), the Solo-1 revision D becomes the Solo-1E with a final pin-out with changes as shown in the table below.  Samples of Solo-1E will be in mid-April 98 and Production targeted for July 98.

For motherboard (and notebook) designs:

(1) Existing designs:

Solo-1 revision D is in full production.  Solo-1E is pin compatible to the Solo-1 revision D.  Existing motherboard designs (and notebook designs, if any) could migrate to Solo-1E without requiring any design changes.

(2) New/Future designs:

Also added to Solo-1E is the I2S interface for MPEG audio.  I2S input shares the music DAC with ESFM and hardware wavetable.  ESFM, HWWT, I2S are mutually exclusive devices.

The pins affected are:

| Solo-1 Rev. D |  |  | Solo-1E |  |
| --- | --- | --- | --- | --- |
| Number | Name | Description | New name | New description |
| Pin#59 | IRQ7 | ISA IRQ | SEDO/IISLR | serial data to EEPROM or I2S LR |
| Pin#58 | IRQ9 | ISA IRQ | IRQ9/SECLK | ISA IRQ or serial EEPROM clock |
| Pin#21 | VOLUP | volume up | VOLUP/SEDI | vol. up or serial data from EEPROM |
| Pin#20 | VOLDN | volume down | VOLDN/SECS | vol. down or serial EEPROM CS |
| Pin#22 | MCLK | MIDI clock | MCLK/IISCLK | MIDI clock or I2S clock |
| Pin#23 | MSD | MIDI serial data | MSD/IISDATA | MIDI data or I2S data |