GEM(R)/3 Driver Pack #2

README.DOC File

March 1988

This file is divided into the following sections:

1.  Hewlett Packard HPGL (and Compatibles) Plotter Configuration File

2.  Houston Instruments DM/PL Plotter Configuration File

______________________________________________________________________

SECTION 1. USING THE HEWLETT PACKARD HPGL (AND COMPATIBLES) PLOTTER

CONFIGURATION FILE

* The HPGL Driver supports a configuration file to allow proper

operation when the driver cannot communicate with the plotter

directly. It may also be used to customize the plotter's

performance with certain pen and media combinations.

* The configuration file MUST be named VDHPX.CNF and reside in the

\GEMAPPS\GEMSYS directory.

* The configuration file will usually be used to slow the plotter

down when making transparencies. For this purpose, the velocity

command (see below) is provided. Follow the manufacturer's

recommendation for the pens and media you will use.

* The plotter should be set up for 9600 baud, 8 data bits, 1 stop bit,

and no parity. Refer to the plotter documentation to see how this

is done.

* There are plotters which do not support the HPGL "OI"command but which

allow the driver to identify the HP plotter model by other means. If

your plotter does not support the HPGL OI function, you may specify

in the configuration file the HP plotter model number that best

matches your plotter.

* When using OUTPUT.APP, changing the paper size has no effect until a

new start command is given. Also, changing paper size in the middle of

an output list is not recommended.

* Commands available for the configuration file.

model(nnnn)   - This identifies the plotter model to the driver.

THIS IS ALSO THE ONLY CONFIG FILE PARAMETER THAT

TAKES PRECEDENCE OVER INFORMATION RETURNED FROM

THE PLOTTER. Using this parameter has the effect

of suppressing an identifier inquiry to the

plotter from the driver. This is done because

some plotters do not support the HPGL OI

command. Acceptable model numbers are:

7440	7470	7475	7550

7570	7580	7585	7586

numpens(n)    - Informs the driver of the number of pens to be

made available. The number of pens used is

determined by the model number specified. The

default is 8.

penwidth(x)   - Specifies pen width in tenths of a millimeter.

The default is 0.3mm.

velocity(v)   - Specifies pen velocity in centimeters/second.

The default is dependent on the model of the

plotter.

roman()	      - No parameters. Its presence in the configuration

file tells the driver that the plotter supports

Hewlett Packard's roman (character set 7)

extensions set.

init(". . .") - Specifies a string of HPGL commands to be sent

to the plotter after it has been initialized.

The double quotes are mandatory. In addition,

the string MUST NOT contain any double quotes.

Control characters, however, are allowed.

final(". . .") - This is similar to the init command, however,

it is sent to the plotter when the plotting

session is being terminated.

_____________________________________________________________________

SECTION 2. USING THE HOUSTON INSTRUMENTS DM/PL PLOTTER

CONFIGURATION FILE

* The DM/PL Driver supports a configuration file to allow proper

operation when the driver cannot communicate with the plotter

directly. It may also be used to customize the plotter's

performance with certain pen and media combinations.

* The configuration file MUST be named VDHIX.CNF and reside in the

\GEMAPPS\GEMSYS directory.

* The configuration file will usually be used to slow the plotter

down when making transparencies. For this purpose, the velocity

command is provided. Follow the manufacturer's recommendation for

the pens and media you will use.

* The plotter should be set up for 9600 baud, 8 data bits, 1 stop bit,

and no parity. Refer to the plotter documentation to see how this

is done.

* When using this plotter with OUTPUT.APP, only one filename should

be entered in an output list at one time. Entering multiple files

will mean that the output for successive copies or files will not

be positioned correctly on the plotter paper.

* The following commands are available for the configuration file.

model(nn)     - Identifies the plotter model to the driver.

Acceptable values are 29, 41, 42, 51, 52, 61,

and 62.

numpens(n)    - Informs the driver of the number of pens to

be made available to the application. The

default is 14.

penwidth(x)   - Specifies pen width in tenths of a millimeter.

The default is 0.3mm.

velocity(v)   - Specifies pen velocity in centimeters/second.

The default is dependent on the model of the

plotter.

chart(x)      - Specifies the chart size.  Use "s" for small

and "l" for large chart format.

resolution(n) - Specifies the resolution for the plotter to use.

Normally,  the default resolution is best.

However, one can obtain some reduction in plot

file size by specifying a coarser resolution.

Use the following values for n to specify

resolution:

Value   Resolution(mm)

1 =  .001

2 =  .005

3 = 0.1

4 = 0.025

The default is 0.025mm.

init(". . .") - Specifies a string of DM/PL commands to be sent

to the plotter after it has been initialized.

The double quotes are mandatory. In addition,

the string MUST NOT contain any double quotes.

However, control characters are allowed.

final(". . .") - This is similar to the init command. It is sent

to the plotter when the plotting session is

being terminated.

End of README.DOC#####################################################################