The 16550A UART

by Randol Tigrett

If your computer includes a 16550A UART, chances are you'll get faster

throughput during your communications sessions if your software takes

advantage of it. To experiment with the UART in your machine, you can

use the DEBUG script shown in Figure 1 to generate a small program

called FIFO.COM. You simply run FIFO.COM once when you turn on your PC

and it activates the FIFO buffers in the UART, displaying what type of

UART you have in your machine, 8250 or 16550 or 16550A.

We emphasize the word experiment because some communications

programs, written before the introduction of the 16550A, might lock up

when the UART activates the previously unused upper bits of the

Interrupt ID Register. Even if your communications software doesn't

lock up with the FIFO buffer activated (and most don't) you still

won't get the full benefit of the FIFO buffer unless the program makes

use of it.

Programs not specifically written for the 16550A won't move

data in large blocks the way more modern programs do, and FIFO.COM

can't make your communications program recognize the UART or utilize

its FIFO buffer better. However, activating the buffer with FIFO.COM

can potentially eliminate problems of overrunning the UART,

particularly in a PC on the receiving side of a high-throughput file

transfer. Although it may help in this way, FIFO.COM mainly

demonstrates how programmers can activate the buffer.

The NS16550AF UART (Universal Asychronous

Receiver/Transmitter) contains two 16-byte FIFO (first in, first out)

buffers. Slower machines, like an 8-MHz AT, using fast modems need

these buffers, as do faster machines, like a 33-MHz 386, that attempt

to run other programs in a multitasking mode while using asynchronous

communications. The interrupts generated by communications sessions

can have a serious impact on the effectiveness of computers running

multitasking operations. Programs in other sessions suspend

processing, displays don't update, and characters typed at the

keyboard don't register while the CPU services the UART.

The 16-byte FIFO buffers reduce the interrupt activity of the

CPU because they can hold more data before generating an interrupt

requesting a transfer. This makes it easier for slower CPUs to avoid

dropping characters when they receive a fast stream of data. You can

set the receive FIFO buffer to receive 1, 4, 8, or 14 characters

before the UART generates an interrupt.

If your computer contains a 16550 UART instead of the 16550A

or AF, be aware that the plain 16550 was flawed and will not work

correctly if the FIFO buffers are activated. When you buy a new

computer, make sure the manufacturer either sockets the UART or

installs the new 16550A UART. If you can install the UART yourself, it

costs only about $12. The 16550A UART is especially useful for

communicating with high-speed modems because, with the advent of

V.32bis modems, you can now enjoy a carrier rate of 14,400 bits per

second between modems. With V.42bis data compressions of 4 to 1, you

must set the DTE (Data Terminal Equipment) speed of the RS-232C link

to 57,600 bps for optimum performance during file transfers. Slower

PCs cannot handle the high RS-232C DTE speed without the 16550A UART.

If you want more information on programming the 16550A UART,

National Semiconductor makes available a book called Data

Communications: Local Area Networks: UARTs. The book sells for $10 and

you can call National Semiconductor at 408-721-5000 to ask about its

availability.

HOW TO GET FIFO.COM

You can download FIFO.COM from library 2 of PC MagNet's Utilforum/Tips

forum (type GO PCM:TIPS from PC MagNet or CompuServe to get there) or

create it using the DOS DEBUG program.

Assuming DEBUG is available in the C: root directory, redirect

the following text to DEBUG by typing

c:\debug <fifo.scr

The text below contains comments to illustrate the program's

operation. Remove all comments enclosed in /* */ before redirecting to

DEBUG. Note that this program will cause the 16550 UART to work

improperly by enabling its flawed FIFO buffer.

A

JMP     0123

NOP

AND     [BX+DI],DH

SS:

XOR     AX,3035

AND     [DI+41],DL

PUSH    DX

PUSH    SP

AND     [BX+SI],AH

AND     [BX+SI],AH

AND     AL,20

CMP     [BP+SI],DH

XOR     AX,2030

PUSH    BP

INC     CX

PUSH    DX

PUSH    SP

AND     [BX+SI],AH

AND     [BX+SI],AH

AND     [SI],AH

MOV     DX,03F8     /* PORT ADDRESS - 0040:0000 */

ADD     DX,+02      /* set offset to IIR Register */

MOV     AL,C1       /* set receive buffer to 14 and enable */

OUT     DX,AL       /* change the 14 = 0x0C to 1,4,or 8 */

XOR     AX,AX

IN      AL,DX

PUSH    AX

AND     AL,C0       /* set up for 16550A UART */

CMP     AL,C0       /* is it a 16550A ?       */

JNZ     013D        /* no                     */

LEA     DX,[0103]   /* yes                    */

JMP     0141

NOP

LEA     DX,[0113]   /* print either 16550 or 8250 */

MOV     AH,09

INT     21

POP     AX

MOV     AH,4C       /* show the IIR setup as errorlevel */

INT     21

RET

N FIFO.COM

RCX

4B

W

Q

Figure 1: DEBUG script to generate FIFO.COM, a program that activates

the 16550A UART's FIFO buffer and identifies the type of UART in your

machine.

===============================================

Toad Hall Note:

The above horribleness can be circumvented by using the new FIFO.ASM

I just produced with a *real* disassembly of FIFO.COM.

And if you want something a wee bit more sophisticated, look at

UARTTYPE.ASM (with its accompanying UARTTYPE.MSG).  It not only activates

the UART, but specifically identifies just which one it is!

David Kirschbaum

Toad Hall

kirsch%maxemail@uunet