====================================================================

Programs:                  SAMPL.EXE & SAMVID.EXE

playing:                   WAV, VOC & raw data

via:                       PC Speaker, COVOX & Sound Blaster.

====================================================================

SAMPL / SAMVID

SAMPL plays digitalized sound  samples  through internal PC Speaker,

COVOX (JDAC) or Sound Blaster & compatibles sound  cards  (also  GUS

via  sbos).   Digitalized sound sample may be in Windows *.WAV file,

CreativeLabs *.VOC file,  or  in  common  raw  data file without any

header.  SAMPL has many command line switches, which may modify  its

playing  parameters.   While playing, there are many active HotKeys.

SAMPL is an text application, while SAMVID is its graphical version.

Looking Past

On market, there are  many  programs  playing  MOD's via PC Speaker,

COVOX and SoundBlaster.  But there are much  less  programs  playing

common  samples  (not MOD's) on any hardware.  Here comes SAMPL, You

can easy "view" your  samples  files,  doesn't matter You have Sound

Card, COVOX or only PC Speaker, doesn't matter Your samples are WAV,

VOC or RAW data, doesn't matter You have 486 or only AT (XT?!).

====================================================================

Keys active in playing time

Enter            - Stop/Play Music

PgUp/PgDown      - Tape Back/Forward

Home             - Tape Home

Left/Right Arrow - Increase/Decrease Speed

Down Arrow       - Default Speed

0..5             - Sound Device

Esc              - Exit

D                - Dos Shell

Devices:

0 - PC Speaker

1 - COVOX in LPT1 (if port exist)

2..4 - COVOX in LPT2..4 (if port exist)

5 - Sound Blaster (if present)

7 - silence (phantom device)

A try to switch to a non existent device do not make a change.

A switch to LPT port with no COVOX in it  goes  OK,  but  of  course

there will be no sound.

====================================================================

Parameters (switches) specified at command line

Usage:    SAMPL sampl.ext [/n]

SAMPL /?  - for info

Output to D/A on: /0 - PC Speaker

/1 - LPT1 (default)

/2 - LPT2

/3 - Proto Card at address 300h

/Pxxx - Port number (20..3FFh)

-No switch for SBlaster (autodetect!)

/Sxxxx - Sample speed in Hz (1000 to 55000)

/Rxx   - Repeat sample xx times (1 to 255)

/Q     - Quiet Mode - No Display

/QH    - No HotKeys Hints Display

/Fxxxx - Carrier Frequency in Hz (1000 to 25000)

/I     - call old clock interrupt int08h

/D     - Dos Shell

/C=command - Execute command

/Bxxxx - Buffer size for sample in bytes (use with /D or /C)

/O     - wide oscilloscope (only in graph version)

Switches can be specified both at command line and in SAMPL.INI file

*

Sampl

Run w/o parameters shows above help screen.

*

Sampl /?

or

Sampl /h

With question mark or "/h" shows short inform.

*

Except  above  examples,  always  in  command  line sample file name

should be specified, and additionally other parameters (switches).

*

In VOC file  case,  specifying  file  name  with question mark gives

information about file structure.

SAMPL filename.VOC /? > info.txt

*

Choice of output device

/0 - PC Speaker, internal computer beeper, default setting

/1 - COVOX in LPT1

/2 - COVOX in LPT2

/3 - D/A Converter in port 300h

/Pxxx - D/A Converter in port xxx (range: 20..3FFh)

-Sound Blaster (or compatible) is  detected  automatically,  and  no

switches are necessary.

Other switches

*

/Sxxxx

Speed (frequency) of sound playing xxx in Hz (range: 1000 to 55000).

Default is /S8007 (8007 Hz)

*

/Rxx

Repeat number xx (range: 1 to 255). Default never-ending loop (/R0).

Switch /R1 is equivalent to /R (repeat one time).

*

/Q

Quiet mode, no display output (except Copyright) for batch mode.

*

/QH

No HotKey list display (less text on screen).

*

/Fxxxx

Frequency  of  sound  carrier  xxx  Hz, hardware PIT programming, be

extremely careful with changing  this  parameter,  too big value may

cause hang  Your  computer.   Default  value  is  /F15909,  on  slow

computers use /F8007.  Value higher than /F22000 may hang system.

*

/I

Call old clock interrupt chain.  By default it is not called because

programs  using  it (disk cashes, TSR's) may cause system hanging or

incorrect SAMPL.EXE work.  Use this option very carefully.

*

/D

Shell to DOS.

*

/C=command

Shell to DOS and run program with a name "command".

*

/Bxxxx

Buffer, digitalized sample length that stay in memory while shelling

to DOS (/D or /C).

Free memory =

Length of program Sampl +

Buffer of sample +

Free memory while shelling to DOS (/D or /C)

*

/O

Wide oscilloscope (only graph version - SAMVID)

====================================================================

Technical notes

- DO NOT turn  "TURBO  SWITCH"  while  playing sample, because SAMPL

tests computer speed at start time  and  adjust  itself  to  machine

parameters.

-  Program  for sound playing uses timer interrupt, and increase its

frequency, but after usage of  /I  switch it simulate standard clock

speed for other programs.

- Program works only under DOS control, there is no  possibility  to

run it in DOS session of Windows or OS/2.

-  It's  recommended,  in  trouble  case, to remove from memory disk

caches like SmartDrive or Norton Cache, and especially to switch off

its delayed write option, because  they use clock interrupt, and may

conflict with SAMPL.

- It's recommended, in trouble case, to turn off in CONFIG.SYS  file

virtual  mode  V86 drivers (EMS memory drivers) like EMM386 or QEMM.

They cause noticeable delay  in  interrupt service.  It causes, that

even fast 386DX, from SAMPL point of view,  goes  like  slow  AT286.

Eventually  it  is possible to decrease carrier frequency to 8000 Hz

(switch: /F8007), what involves  sound  quality loose.  It should be

said, that QEMM work better with SAMPL, then other memory drivers.

- Sorry, DOS Shell option ( /D or /C at command line, or key "D"  at

run  time)  doesn't  work  properly  with some sound cards, so it is

recommended to use it only with PC Speaker or COVOX.

- Some  options,  naturally,  work  only  with  program text version

(SAMPL), and other only with graph version (SAMVID).

====================================================================

Call without parameters - list of switches

C:\=>sampl

SamplePlayer for Multimedia  ver. 1.0

PCSpeaker, COVOX and SoundBlaster Supported!!!

(C) Przemyslaw Osocha  SKAWINA 1992-94

Usage:    SAMPL sampl.ext [/n]

SAMPL /?  - for info

Output to D/A on: /0 - PC Speaker (default)

/1 - LPT1

/2 - LPT2

/3 - Proto Card at address 300h

/Pxxx - Port number (20..3FFh)

-No switch for SBlaster (autodetect!)

/Sxxxx - Sample speed in Hz (1000 to 55000)

/Rxx   - Repeat sample xx times (1 to 255)

/Q     - Quiet Mode - No Display

/QH    - No HotKeys Hints Display

/Fxxxx - Carrier Frequency in Hz (1000 to 25000)

/I     - call old clock interrupt int08h

/D     - Dos Shell

/C=command - Execute command

/Bxxxx - Buffer size for sample in bytes (use with /D or /C)

/O     - wide oscilloscope (only in graph version)

Switches can be specified both at command line and in SAMPL.INI file

C:\=>

====================================================================

Call with question mark switch - information

C:\=>sampl /?

SamplePlayer for Multimedia  ver. 1.0

PCSpeaker, COVOX and SoundBlaster Supported

(C) Przemyslaw Osocha  SKAWINA 1992-95

Program  plays  samples  on  digit2analog  converter:

COVOX, a simple device connecting Your PC &  Your  Hi-Fi.

You  can buy it or made yourself, it is very easy.  COVOX

contain 8 resistors and  plug  for  LPT port.  It receive

digital signal from LPT port.

>>>  Now also plays on PC Speaker and Sound Blaster.  <<<

This  is  freeware  version,  You  can  use & copy it

w/o  any  limitations!

This  program  is  under continuous development.  If

You have  any  suggestions  or  questions, please contact

directly me:

Przemyslaw Osocha

ul.Ogrody 34

32-050 Skawina

POLAND

E-mail: osocha@mech.pk.edu.pl

C:\=>

====================================================================

Call with sample file name - playing

C:\=>sampl sound.voc

SamplePlayer for Multimedia  ver. 1.0

PCSpeaker, COVOX and SoundBlaster Supported!!!

(C) Przemyslaw Osocha  SKAWINA 1992-94

Enter            - Stop/Play Music

PgUp/PgDown      - Tape Back/Forward

Home             - Tape Home

Left/Right Arrow - Increase/Decrease Speed

Down Arrow       - Default Speed

0..5             - Sound Device

Esc              - Exit

D                - Dos Shell

MemAvail: 516288, FileLen:  45900

Playing: SOUND.VOC

Device : PC Speaker

Position:     3222,    Ratio:  7 %,    Speed: 10989 Hz

������������������������������������������������������������������ͻ

������������������������������������������������������������������ۺ

������������������������������������������������������������������ͼ

C:\=>

====================================================================

DISCLAIMER

Presented package is FreeWare.

That's mean , You can use it, and also copy  for  Yourself  and  for

others, without any fee for author.  You have not to send any money,

any  postcards  or anything else.  Package is fully working version,

without any  time  limits  (no  evaluation)  and  without non active

functions (no cropped).

Distribution is only possible for  full  package,  so  for  program,

documentation and other files together, with no any changes.

Package  have not to be sold for profits, it can be distributed only

for free, without  any  profits.   The  only disclaimer is shareware

house idea, where fee can be charged for  distributions  costs.   In

case  of  package  inclusion into other package, that is distributed

for profits there must be  an  author  written permission for such a

distribution.

Package can be freely used, but there are not allowed any changes or

debugging.  The package IS NOT Public Domain.  So: You have  not  to

change  Copyright  string  in binary code, have not to change binary

code or data in  it.   No  allowed  reverse engineering.  No allowed

changes in included documentation or data  files.   Any  changes  or

added  files must be exactly tagged, that they are not original part

of package.

I can not guarantee, that  I'll  answer  to any letter, or that I'll

make any suggested changes to program code, because this package  is

distributed for free and I have no time waste on that.  This program

is  distributed  on rule AS IS.  But all letters containing detected

errors  descriptions  or   suggested   innovations   will  be  nice.

Disclaimer:

In no event the Authors will be liable for  any  damages,  including

any  lost profits, lost savings or other incidental or consequential

damages arising out of your use  or inability to use the program, or

for any claim by any other party.

All mentioned Trademarks and  Copyrights  are  registered  by  their

respective owners.

====================================================================

LAST WORD

In  malfunction  case, please give me detailed situation description

including co-working software list.   It  will  be nice also, to see

Your suggestions for farther program development.

All notes, suggestions, proposal are welcome.

Przemyslaw Osocha

ul.Ogrody 34

32-050 Skawina

POLAND

E-mail: osocha@mech.pk.edu.pl

--------------------------------------------------------------------

<EOF>